KELVIN6K ROBOTICS & AUTOMATION | TECHNICAL DOCUMENTATION

Kelvin6K RAG Knowledge Dataset & Technical Inventory

Comprehensive Operational Reference, Troubleshooting Procedures, and Catalog of 358 Verified Voice Question Prompts

| System Name | Kelvin6K 3D Concrete Printing Voice Assistant |
| --- | --- |
| Documentation Scope | Full Inventory of All Ingested Knowledge Chunks & Procedures |
| Source Documents | 9 Documents (7 in docs/knowledge/, 2 in docs/) |
| Total Indexed Chunks | 101 Chunks across 9 Files |
| Question Prompts Cataloged | 358 Questions (203 English, 155 Hinglish) |
| Troubleshooting & SOP Records | 92 Operational & Diagnostic Records (84 Q&A + 8 Manual SOPs) |
| Index Storage & Mode | rag_index/index.json ¦ Lexical BM25 (Deterministic Keyword Scorer) |
| Status & Verification | Active Production Knowledge Base - Zero Hallucinations Policy |

1. Executive Summary & RAG System Metrics

This report provides an exhaustive, verified technical inventory of the Retrieval-Augmented Generation (RAG) knowledge base currently active in the Kelvin6K Construction Robotics Voice Assistant platform. The knowledge base serves as the deterministic source of truth for site operators, technicians, and automated monitoring agents operating large-format 3D concrete extrusion equipment.

| Metric / Parameter | Current Value | Operational Significance |
| --- | --- | --- |
| Total Source Documents | 9 Documents | 7 specialized knowledge files + 2 core system manuals |
| Total Indexed Chunks | 101 Chunks | 100% of source files chunked and indexed in rag_index/index.json |
| Total Question Prompts | 358 Questions | Authored questions embedded directly into indexed chunks |
| English Questions | 203 Questions | Standard technical terminology and conversational English phrasing |
| Hinglish Questions | 155 Questions | Phonetic bilingual site phrasing (e.g. 'clean kab karna hai?') |
| Total Solutions & Procedures | 92 Records | 78 explicit solution fields + 14 manual & diagnostic procedures |
| Explicit Q&A Records | 84 Records | Structured records featuring Problem, Cause, Solution, & Questions |

2. Complete Knowledge Document Directory

| Document Filename | Storage Location | Chunks | Records | Questions | Primary Subject Matter |
| --- | --- | --- | --- | --- | --- |
| conflicts.md | docs/knowledge/ | 7 | 6 | 29 (17/12) | Conflicting site vs manual values needing confirmation |
| machine_parameters.md | docs/knowledge/ | 3 | 2 | 11 (7/4) | Observed UI screen snapshot & layer cycle timing |
| operational_rules.md | docs/knowledge/ | 7 | 6 | 41 (24/17) | Mandatory cleaning intervals & operational checklists |
| printing_issues.md | docs/knowledge/ | 14 | 13 | 64 (36/28) | Project-derived PRINT-series issues (PRINT-001..023) |
| safety_rules.md | docs/knowledge/ | 4 | 3 | 15 (9/6) | Emergency stop boundaries, PPE, and temperature rules |
| site_incidents.md | docs/knowledge/ | 51 | 50 | 182 (100/82) | 50 verified site breakdowns from WhatsApp chat logs |
| visual_defects.md | docs/knowledge/ | 5 | 4 | 16 (10/6) | Visual camera-observed defects & void classifications |
| kelvin6k_manual.md | docs/ | 4 | 3 | SOP (0/0) | Standard procedures for clog, calibration, maintenance |
| kelvin6k_troubleshooting.md | docs/ | 6 | 5 | Diag (0/0) | Diagnostic guide for delamination, shift, bonding, flow |

3. Detailed Document Profiles & Technical Scope

3.1 conflicts.md

| File Path & Chunks | docs/knowledge/conflicts.md ¦ 7 chunks (1 preamble + 6 records) |
| --- | --- |
| Purpose & Scope | Documents critical discrepancies where site observations conflict with written specifications. Enforces strict 'MUST CONFIRM, DO NOT ASSUME' protocol. |
| Provenance / Authority | HIGH-PRIORITY WARNING / DUAL-VALUE REPORTING. Assistant must state both values to avoid machine or structural failure. |
| Records Covered | 6 records covering: 40 mm vs 50 mm layer thickness, 3 vs 5 layer fly ash fill, 150% feedrate dual-meaning, Z-height multiple corrections, sand compaction vs wet layers fallen wall cause, and 600 layer context-specificity. |
| Supported Questions | 17 English, 12 Hinglish (29 total). Supported inquiries address specification confusion and ambiguous parameters. |

3.2 machine_parameters.md

| File Path & Chunks | docs/knowledge/machine_parameters.md ¦ 3 chunks (1 preamble + 2 records) |
| --- | --- |
| Purpose & Scope | Records screen snapshots and observed cycle times taken from an actual print run. Distinguishes observed UI values from universal standards. |
| Provenance / Authority | OBSERVATION ONLY. Sourced from 3D_Concrete_Printing_Agent_Training_Dataset(2).pdf sections 1 & 2. |
| Records Covered | 2 records covering: Machine UI parameter screen readouts (feedrate, travel speed, layer counters) and observed 1m 30s layer cycle times. |
| Supported Questions | 7 English, 4 Hinglish (11 total). Supported inquiries address screen readings, cycle times, and layer metrics. |

3.3 operational_rules.md

| File Path & Chunks | docs/knowledge/operational_rules.md ¦ 7 chunks (1 preamble + 6 records) |
| --- | --- |
| Purpose & Scope | Establishes mandatory cleaning intervals, routine maintenance timing, pre-print planning checklists, and assistant diagnostic rules. |
| Provenance / Authority | SITE-VERIFIED & DOCUMENTED. Sourced from 3D_Printing_Issues_Solutions_AI_Agent_Dataset_No_Atom_V2(3).pdf and PRINT-021. |
| Records Covered | 6 records covering: 4 cleaning intervals (hole, pipe, hopper, pump), inspection timing, pre-print verification, during-print handover, and diagnostic response rules. |
| Supported Questions | 24 English, 17 Hinglish (41 total). Supported inquiries address maintenance intervals, cleaning schedules, and pre-print rules. |

3.4 printing_issues.md

| File Path & Chunks | docs/knowledge/printing_issues.md ¦ 14 chunks (1 preamble + 13 records) |
| --- | --- |
| Purpose & Scope | Captures the project-derived PRINT-series troubleshooting catalog generated from manual and monitoring alert rules. |
| Provenance / Authority | DOCUMENTED (Project-Derived). Root causes are marked as LIKELY hypotheses requiring operator verification. |
| Records Covered | 13 records covering: PRINT-001 through PRINT-023 (low material flow, height deviation, pump stopped, accelerator stopped, camera errors, jumpy status, missing layers, pipe cleaning, coupling maintenance, pump seals). |
| Supported Questions | 36 English, 28 Hinglish (64 total). Supported inquiries address alarms, flow anomalies, sensor errors, and mechanical faults. |

3.5 safety_rules.md

| File Path & Chunks | docs/knowledge/safety_rules.md ¦ 4 chunks (1 preamble + 3 records) |
| --- | --- |
| Purpose & Scope | Defines core machine and physical safety rules that take priority over all other answers. Establishes the advisory-only boundary of the assistant. |
| Provenance / Authority | MANDATORY SAFETY POLICY. The assistant must never claim to have remotely changed physical machine settings. |
| Records Covered | 3 records covering: Machine safety boundaries, temperature safety thresholds, and site PPE compliance (helmets, safety footwear). |
| Supported Questions | 9 English, 6 Hinglish (15 total). Supported inquiries address emergency stops, hot/cold weather limits, and PPE rules. |

3.6 site_incidents.md

| File Path & Chunks | docs/knowledge/site_incidents.md ¦ 51 chunks (1 preamble + 50 records) |
| --- | --- |
| Purpose & Scope | Comprehensive operational log of 50 real-world site breakdowns and actual corrective actions taken during field operations. |
| Provenance / Authority | SITE-VERIFIED. Extracted from 1,044 lines of WhatsApp field communications (Aug 31 - Sep 6, 2026). |
| Records Covered | 50 detailed incident records covering: Hose chokes, stone in rotor, cable failures, Z-axis servo fault E08.5, DC startup error, rain stoppages, sun exposure, and material hardening. |
| Supported Questions | 100 English, 82 Hinglish (182 total). Supported inquiries address specific mechanical, electrical, and operational breakdowns. |

3.7 visual_defects.md

| File Path & Chunks | docs/knowledge/visual_defects.md ¦ 5 chunks (1 preamble + 4 records) |
| --- | --- |
| Purpose & Scope | Catalog of visual print defects observed via camera or operator inspection. Provides standard defect terminology and preliminary checks. |
| Provenance / Authority | OBSERVATION ONLY (Unconfirmed). Root causes and solutions must be verified with operator/technician. |
| Records Covered | 4 records covering: Layer gaps/voids, layer deformation/irregular deposition, vertical/through-layer cracks, and fine surface crazing. |
| Supported Questions | 10 English, 6 Hinglish (16 total). Supported inquiries address visual surface flaws, cracking, and bead irregularities. |

3.8 kelvin6k_manual.md

| File Path & Chunks | docs/kelvin6k_manual.md ¦ 4 chunks (1 title preamble + 3 procedural chunks) |
| --- | --- |
| Purpose & Scope | The foundational operating manual defining primary standard operating procedures (SOPs). |
| Provenance / Authority | OFFICIAL SYSTEM MANUAL. Preserves standard factory procedures. |
| Records Covered | 3 core procedures: Nozzle Clog Procedure (solvent soak & cleaning rod), Layer Width Calibration (PIXELS_PER_MM), and Daily Maintenance. |
| Supported Questions | Procedural reference for nozzle clearing, camera calibration, and 4-hour pipe cleaning schedules. |

3.9 kelvin6k_troubleshooting.md

| File Path & Chunks | docs/kelvin6k_troubleshooting.md ¦ 6 chunks (1 title preamble + 5 diagnostic chunks) |
| --- | --- |
| Purpose & Scope | Defines symptom-cause-remedy relationships for common extrusion and geometric defects. |
| Provenance / Authority | OFFICIAL DIAGNOSTIC GUIDE. Factory troubleshooting standards. |
| Records Covered | 5 diagnostic topics: Delamination, Layer Shifting, Poor Layer Bonding, Over-extrusion (wide layers), and Under-extrusion (narrow layers). |
| Supported Questions | Diagnostic reference for layer bonding failures, geometric drift, and material flow imbalances. |

4. Major Knowledge Categories & Taxonomy

Based strictly on the content of the 9 ingested documents, the Kelvin6K RAG knowledge base is organized into eleven operational categories. Concepts related to thermoplastic FDM 3D printing (such as bed warping, heated bed adhesion, or filament retractions) are completely absent, reflecting Kelvin6K's identity as a heavy construction concrete printing system.

• 1. Concrete Delivery & Material Flow: Hose choking and clogging, stones jamming the stator/rotor, raw sand screening and filtration, material hardening during unexpected pauses, batch aging, hopper motor overheating, and material shortage.

• 2. Nozzle, Extrusion & Chemical Dosing: Nozzle clogs, solvent soak and cleaning rod procedures, accelerator hole blockage and big Allen key clearing, chemical dosing pump failures, and extrusion width abnormalities (over- and under-extrusion).

• 3. Layer Quality & Inter-layer Bonding: Delamination between layers, layer shifting due to motor step skipping or green strength failure, cold joints and poor inter-layer bonding, uneven layer pitch, and missing deposition passes.

• 4. Visual & Structural Surface Defects: Camera-observed layer gaps and voids, irregular bead deposition, vertical through-layer cracks, fine surface crazing, curved or arc wall distortion, and front elevation print flaws.

• 5. Foundation & Physical Wall Stability: Base sand compaction failures, structural instability of green bottom layers, wall collapse caused by wet layer overload, lifting damage during crane rigging, and U-joint assembly issues.

• 6. Electrical, Motion & Robotic Hardware Faults: Z-axis servo fault E08.5, Distributed Clock (DC) EtherCAT startup errors, J1 Ethernet cable failures, micro-USB Type-A cable disconnections, VFD wiring issues, loose/open wiring, and E-axis mixer switch/chain failures.

• 7. Operational Protocols & Maintenance Schedules: Mandatory cleaning intervals (hole: 5 layers; pipe: 4 hours; hopper: each wall; pump front: 2 walls), inspection cycles, pre-print planning checklists, and shift handover protocols.

• 8. Machine UI Parameters & Cycle Timing: Screen snapshot readouts, travel speeds, feedrate overrides, layer counter tracking, contextual 1m 30s layer cycle times, and 600 total layer contextual boundaries.

• 9. Conflicting Specifications & Unverified Standards: Explicit documentation conflicts requiring operator confirmation: 40 mm vs 50 mm layer thickness, 3 vs 5 layer fly ash fill depth, dual meanings of 150% feedrate, and multi-source Z-height correction methods.

• 10. Site Environmental Controls & Personnel Safety: Mandatory personal protective equipment (helmets, safety footwear), ambient temperature thresholds, rain interruptions and site shelter, direct sun exposure on pump, and crane/tent clearances.

• 11. Vision Metrology & Camera Monitoring: Layer width calibration using reference blocks and PIXELS_PER_MM, camera disconnections, jumpy tracking status, inactive camera alerts, and lighting/network interference.

5. End-to-End RAG Pipeline Architecture & Component Status

The Kelvin6K RAG architecture is implemented entirely within agent/rag/ and agent/spoken.py. It follows a strict pipeline: Ingestion -> Chunking -> Indexing -> Retrieval -> speak_knowledge -> Spoken Formatter. The following table details the operational status, implementation file, and exact functionality of each component.

| Pipeline Stage | Implementation File | Status | Functional Description & Behavior |
| --- | --- | --- | --- |
| 1. Ingestion | agent/rag/ingest.py ingest_docs.py | OPERATIONAL | Loads .md, .txt, .pdf documents. Extracts page numbers from PDFs and identifies document types. |
| 2. Chunking | agent/rag/ingest.py chunk_text() | OPERATIONAL | Splits on Markdown #/## headings first. Sections >800 chars are windowed with 100 char overlap at paragraph/sentence boundaries. |
| 3. Indexing | agent/rag/store.py agent/rag/embeddings.py | OPERATIONAL (Lexical BM25) | Persists chunks in rag_index/index.json. Active mode is lexical-bm25 (offline deterministic BM25 scorer without external vector dependencies). |
| 4. Retrieval | agent/rag/retriever.py search_documentation() | OPERATIONAL | Tokenizes query, removes stopwords, scores chunks via BM25, and filters hits below RAG_MIN_SCORE (0.30). Returns found=false on miss. |
| 5. Knowledge Selection | agent/spoken.py speak_knowledge() | OPERATIONAL | Iterates over retrieved hits, bypassing empty or preamble chunks (Chunk 0) until the first actionable procedural record is resolved. |
| 6. Spoken Formatter | agent/spoken.py speak_record() & _strip_md | OPERATIONAL | Extracts Solution, Safety, and Cause. Normalizes underscores to spaces for TTS, strips Source: metadata, and clips to max 3 operational sentences. |

6. Question Categories Supported by Kelvin6K

Kelvin6K supports five operational question styles across English and Hinglish inquiries. Because Lexical BM25 query stopword filtering drops conversational pronouns and interrogatives ('how', 'what', 'kitna', 'kyun'), retrieval focuses purely on technical tokens (e.g. 'accelerator', 'clean', 'hose', 'stone', 'E08.5').

• 1. Procedural Instructions ('How do I clean / clear...'): Operator asks for step-by-step physical actions (e.g. cleaning the accelerator hole with an Allen key, soaking a nozzle in solvent, clearing a stone in the stator). Kelvin6K answers with the immediate physical action.

• 2. Maintenance Schedule Queries ('How often / When is cleaning due?'): Operator asks about operational time or layer frequencies. Kelvin6K provides the exact numerical interval (5 layers, 4 hours, 1 wall, 2 walls) and warns against confusing parts.

• 3. Fault Diagnosis & Root Cause ('Why is material not coming out / Why the error?'): Operator asks why a machine fault occurred. If the operator explicitly includes cause-seeking words ('why', 'reason', 'kyun'), the spoken formatter includes the cause and confidence level.

• 4. Specification Clarification & Conflict Queries ('What is the layer thickness?'): Operator asks about values that have conflicting documentation. Kelvin6K states both values clearly (e.g. 40 mm manual vs 50 mm site observation) and instructs the operator to verify before proceeding.

• 5. Safety Boundaries & Operational Limits ('Can we print in the rain?'): Operator asks about environmental conditions or safety rules. Kelvin6K provides immediate precautionary commands (stop printing, cover equipment, advisory-only boundary).

7. Complete Catalog of All 358 Question Prompts & Operational Solutions

The following catalog presents every single structured troubleshooting record and manual procedure across all 9 documents in the Kelvin6K knowledge base. Each record displays its Problem, Symptoms, Cause, Solution / Corrective Action, Safety / Parameters, Verification Status, and all authored English and Hinglish Question Prompts.

Section 7.1: Operational Rules & Maintenance Intervals (operational_rules.md)

Record #1: Kelvin6K Operational Rules and Maintenance Intervals Procedural Content: Sources: 3D_Printing_Issues_Solutions_AI_Agent_Dataset_No_Atom_V2(3).pdf ("Operational Rules Extracted from the Chat") and Kelvin6K_Printing_Issues_and_Solutions_Dataset(2).pdf section 3 ("Agent Response Logic").

Record #2: Issue: Cleaning Intervals - Accelerator Hole, Pipe, Hopper and Pump Front Problem: Four different parts are cleaned on four different intervals, counted in different units. They sound similar and are easily confused. Symptoms / Observations: Material flow becomes less reliable; print quality drifts; a maintenance reminder appears. Cause / Confidence: Overdue routine cleaning. Safety / Important Notes: Do NOT use wire to clean the accelerator hole. If the machine must be stopped for that cleaning, stop in the column area and keep the stop under five seconds. The accelerator HOLE (5 layers, Allen key) and the printing PIPE (4 hours) are different tasks with different tools - do not substitute one for the other. Important Parameters: 5 layers - accelerator hole. 4 hours - printing pipe. 1 wall - hopper. 2 walls - pump front. Verification Status: SITE-VERIFIED for the layer and wall intervals; the 4-hour pipe interval is DOCUMENTED and consistent with the operating manual. Source: PDF 4 records 34, 49; PDF 3 PRINT-021. English Question Prompts: [Q1] How often do I clean the accelerator hole? [Q2] How often do I clean the printing pipe? [Q3] When do I clean the hopper? [Q4] When do I clean the front of the pump? [Q5] What cleaning is due? Hinglish Question Prompts: [Q6] Accelerator hole kitni layers ke baad clean karna hai? [Q7] Pipe kitne ghante mein clean karna hai? [Q8] Hopper kab clean karna hai?

Record #3: Issue: Inspection and Timing Intervals - Coupling, Seals, Pump Running, Restart and Curing Problem: Operators need to know the time-based inspection and operating intervals. Symptoms / Observations: Overdue inspection; material hardening during pauses; long restart gaps; walls not cured. Cause / Confidence: Overdue inspection or an interval not being observed. Safety / Important Notes: One hour is repeatedly stated in the source as the maximum restart window. Kelvin6K has no maintenance history table yet, so it can state these intervals but cannot say whether a task is actually due. Important Parameters: Daily - accelerator coupling. Weekly - pump seals. 5 minutes - pump run during pauses. 1 hour - maximum restart window. 3 times per day - curing. Verification Status: SITE-VERIFIED for the pump-run, restart and curing intervals; DOCUMENTED (project-derived) for the daily and weekly inspections. Source: PDF 4 records 14, 39, 49; PDF 3 PRINT-022, PRINT-023. English Question Prompts: [Q9] How often are pump seals checked? [Q10] Has the accelerator coupling been checked today? [Q11] How often should walls be cured? [Q12] What is the maximum restart time? [Q13] How often should the pump run during a pause? Hinglish Question Prompts: [Q14] Pump seal kab check karna hai? [Q15] Coupling roz check karni hai? [Q16] Curing din mein kitni baar karni hai? [Q17] Restart kitni der mein karna hai?

Record #4: Issue: Pre-Print Verification and Planning Rules Problem: Repeated stoppages and wrong prints were traced to planning and verification gaps rather than to the machine. Symptoms / Observations: Wrong files printed, unclear next wall, missing consumables, no space to print. Cause / Confidence: Process and planning gaps. Solution / Corrective Action: The site rules for before a print starts: Safety / Important Notes: Do not start printing without a sieve where required. Important Parameters: A verified sequence of at least 20 prints was requested. Target discussed: normally 2 walls per shift, recorded as discussed rather than confirmed. Verification Status: SITE-VERIFIED practice. Source: PDF 4, Operational Rules Extracted from the Chat. English Question Prompts: [Q18] What should I check before printing? [Q19] How do I prepare the base? [Q20] What planning is needed before a print? [Q21] Do I need a sieve? Hinglish Question Prompts: [Q22] Printing se pehle kya check karna hai? [Q23] Base kaise taiyaar karna hai? [Q24] Sieve lagana zaroori hai?

Record #5: Issue: During-Print, Restart and Handover Rules Problem: Time was lost between walls, and quality was compromised by prioritising speed. Symptoms / Observations: Long restart gaps; defects accepted on elevation walls; unclear handover. Cause / Confidence: Process gaps during and after printing. Solution / Corrective Action: The site rules for during and after a print: Safety / Important Notes: One hour is repeatedly stated in the source as the maximum restart window. Important Parameters: 1 hour maximum restart window. Verification Status: SITE-VERIFIED practice. Source: PDF 4, Operational Rules Extracted from the Chat. English Question Prompts: [Q25] How long should the restart take? [Q26] What must I do at shift handover? [Q27] Should I prioritise speed or quality? [Q28] What do I record after fixing a problem? Hinglish Question Prompts: [Q29] Restart kitni der mein karna hai? [Q30] Shift handover mein kya batana hai? [Q31] Speed ya quality, kya important hai?

Record #6: Issue: Diagnostic Response Logic for the Assistant Problem: A reported symptom can match several distinct issues, and answering with the wrong one wastes the operator's time or is unsafe. Symptoms / Observations: An operator reports a symptom without enough detail to identify the issue. Cause / Confidence: Overlapping symptoms across distinct issues. Solution / Corrective Action: The documented response logic: Safety / Important Notes: Layer Gap or Void, Delamination, and Poor Layer Bonding can overlap in symptoms but are not identical. Keep the records distinct rather than merging them. Important Parameters: None. Verification Status: DOCUMENTED GUIDANCE for the assistant's behaviour. Source: PDF 3 section 3; PDF 1 section 3.2. English Question Prompts: [Q32] How should the assistant diagnose a problem? [Q33] What questions should be asked first? [Q34] How do I narrow down a printing fault? Hinglish Question Prompts: [Q35] Problem kaise pata karein? [Q36] Pehle kya poochna chahiye?

Record #7: Issue: Cause Confidence and Measurement Reporting Rules Problem: Presenting a guess as a confirmed diagnosis, or a camera inference as machine fact, misleads the operator. Symptoms / Observations: A possible cause being stated as the cause; a pixel measurement being reported as millimetres. Cause / Confidence: Not applicable - these are standing rules. Solution / Corrective Action: The documented reporting rules: Safety / Important Notes: The data review document marks the possible-versus-confirmed distinction as the most important dataset issue, and as MUST FIX BEFORE PRODUCTION USE. Important Parameters: None. Verification Status: DOCUMENTED GUIDANCE for the assistant's behaviour. Source: PDF 1 sections 3.1 and 3.10; PDF 3 section 3. English Question Prompts: [Q37] Is that the confirmed cause? [Q38] Can the camera tell me the machine state? [Q39] Why is the layer width not in millimetres? Hinglish Question Prompts: [Q40] Ye confirm cause hai? [Q41] Camera se machine ka status pata chalta hai? [Q42] Layer width mm mein kyun nahi aa rahi?

Section 7.2: Conflicting Values & Ambiguous Specifications (conflicts.md)

Record #8: Kelvin6K Conflicting Values - MUST CONFIRM, DO NOT ASSUME Procedural Content: These are places where the source documents disagree, or where a value is context-specific and has been mistaken for a standard. None of them is resolved here. Kelvin6K_Printing_Data_Review_Problems_Solutions.pdf sections 3.3, 3.4, 3.5 and 3.6 marks these as MUST CONFIRM, CONTEXT-SPECIFIC or OBSERVATION ONLY. When a question touches any of these, the assistant must state both values and ask which print or setting is meant. It must NOT pick one.

Record #9: Issue: Layer Thickness - Conflicting Values 40 mm and 50 mm Problem: The available data contains different layer-thickness values in different contexts. No universal production standard is established. Symptoms / Observations: Layers reported as too thick in one context; U-joints between walls failing to join in another. Cause / Confidence: Context-dependent. In the U-joint case, 40 mm overlapped the adjacent layer. In the separate too-thick case, 40 mm was reported alongside a 120 mm distance. Solution / Corrective Action: The dataset contains both 40 mm and 50 mm values in different contexts. Confirm the standard value for the active print and settings before changing anything. Safety / Important Notes: Do not state either value as the universal standard. Ask which print and which joint before advising. Important Parameters: 40 mm, 50 mm, plus 15 mm suggested, 120 mm distance. CONFLICTING - none confirmed as the standard. Verification Status: CONFLICT - MUST CONFIRM WITH OPERATOR. Both values are site-reported in different contexts. Neither is confirmed as standard. Source: PDF 4 records 26 and 29; flagged in PDF 1 section 3.3. English Question Prompts: [Q43] What layer thickness should I use? [Q44] Is 40 mm or 50 mm correct? [Q45] What is the standard layer thickness? [Q46] Layer thickness 40 vs 50 mm.? Hinglish Question Prompts: [Q47] Layer thickness kitni honi chahiye? [Q48] 40 ya 50 mm? [Q49] Standard thickness kya hai?

Record #10: Issue: Fly Ash Fill Depth - Conflicting Values 3 Layers and 5 Layers Problem: Two site records give different fly ash fill depths. Symptoms / Observations: Fly ash used for bottom-layer filling and defect correction, with different depths recorded. Cause / Confidence: Context-dependent, or a change in practice between the two records. Not resolved in the sources. Solution / Corrective Action: Both instructions appear in the site records: Safety / Important Notes: Both records say "as instructed", suggesting the depth is decided per wall rather than fixed. Important Parameters: 3 layers and 5 layers. CONFLICTING - neither confirmed as standard. Verification Status: CONFLICT - MUST CONFIRM WITH OPERATOR. Source: PDF 4 records 29 and 30. English Question Prompts: [Q50] How many layers of fly ash? [Q51] Do I fill three layers or five layers with fly ash? [Q52] Fly ash depth.? Hinglish Question Prompts: [Q53] Fly ash kitni layers tak bharna hai? [Q54] Teen ya paanch layers? [Q55] Fly ash kahan dalna hai?

Record #11: Issue: Feedrate - 150 Percent Meaning Two Different Things Problem: The value 150 percent appears in two documents with two different meanings, and 170 percent was rejected. Symptoms / Observations: Elevation print quality unacceptable at 170 percent. Cause / Confidence: Not a fault - a documentation ambiguity. Safety / Important Notes: Quality takes priority over speed for elevation walls. Important Parameters: 170 percent rejected for elevation. 150 percent recorded after correction. 150 percent also observed as a live snapshot reading. Related recorded value 3000 mm/min during arc troubleshooting. Machine limits observed: Max Velocity 6000, Max Rapid 6000, Current Feedrate 3600. Verification Status: CONFLICT - MUST CONFIRM. 170 percent rejection is site-verified; the approved standard is NOT CONFIRMED. Source: PDF 4 record 11; PDF 2 section 2; flagged in PDF 1 section 3.6. English Question Prompts: [Q56] What feedrate should I use? [Q57] Is 150 percent the maximum? [Q58] Is 170 percent too fast? [Q59] What is the approved speed? Hinglish Question Prompts: [Q60] Feedrate kitna rakhna hai? [Q61] 150 percent maximum hai? [Q62] Speed kitni set karni hai?

Record #12: Issue: Z-Height Correction - Multiple Recorded Approaches Problem: The source records more than one approach to the same Z-height problem, and disagrees about whether it is a Z-offset or a G-code issue. Symptoms / Observations: A print stopped with approximately 80 mm remaining because the Z-height limit was not set correctly. Cause / Confidence: Recorded as linked to removal of the Z motor. One message identifies it as a Z-offset issue rather than necessarily a G-code error. Solution / Corrective Action: The source records these, without stating which is correct: Safety / Important Notes: MACHINE-SPECIFIC PROCEDURE - AUTHORIZATION REQUIRED. These are controlled procedures, not generic troubleshooting instructions. Do not issue any of these commands without authorization and without identifying the exact machine condition. Important Parameters: Approximately 80 mm remaining. G92.1, G92 Z0, G92 Z1515. CONFLICTING approaches. Verification Status: CONFLICT within a single site record. AUTHORIZATION REQUIRED. Source: PDF 4 record 25; flagged in PDF 1 section 3.8. English Question Prompts: [Q63] How do I correct the Z height? [Q64] Which G92 command should I use? [Q65] Is this a Z-offset or a G-code problem? Hinglish Question Prompts: [Q66] Z height kaise theek karein? [Q67] Kaunsa G92 command use karna hai?

Record #13: Issue: Fallen Wall Cause - Sand Compaction or Wet Layers Problem: Two records attribute a fallen wall to different causes. Symptoms / Observations: A printed wall fell. A related base defect was attributed to bottom sand compaction. Solution / Corrective Action: Both records agree on the checks: inspect CCTV and layer condition; ensure layers are sufficiently dry before proceeding; and prepare the base correctly by levelling the sand surface and placing polythene before printing. Safety / Important Notes: Do not state either cause as confirmed. Investigate wall instability with CCTV and physical inspection. Important Parameters: None numeric. Verification Status: CONFLICT - CAUSE DISPUTED, neither confirmed. Source: PDF 4 records 30 and 31. English Question Prompts: [Q68] Why did the wall fall? [Q69] Was it the sand or the wet layers? [Q70] What causes a wall to collapse? Hinglish Question Prompts: [Q71] Wall kyun gir gayi? [Q72] Sand ki galti thi ya layers geeli thi? [Q73] Wall collapse ka kya reason hai?

Record #14: Issue: Total Layer Count 600 Is Not a Standard Problem: An observed total of 600 layers has been mistaken for a universal wall layer count. Symptoms / Observations: The machine screen showed 115 completed layers plus 485 remaining, totalling 600. Cause / Confidence: Not applicable - this is a context-specific reading. Solution / Corrective Action: Treat 600 as belonging to that specific print job only. The arithmetic is internally consistent, but it must not be treated as a universal wall layer count. Read the layer count for the active job from live machine telemetry, which is not yet connected to Kelvin6K. Safety / Important Notes: Kelvin6K's database currently holds a different total-layer value for its seeded job. Do not reconcile the two by assumption. Important Parameters: 115 completed, 485 remaining, 600 total - for one specific print job only. Related: the observed layer time of approximately 1 minute 30 seconds is a SINGLE OBSERVATION and not a standard layer time. Verification Status: CONTEXT-SPECIFIC VALUE - NOT a standard. MUST CONFIRM per job. Source: PDF 2 section 2; flagged in PDF 1 sections 3.4 and 3.5. English Question Prompts: [Q74] How many layers are in a wall? [Q75] Is 600 layers standard? [Q76] What is the total layer count? Hinglish Question Prompts: [Q77] Ek wall mein kitni layers hoti hain? [Q78] 600 layers standard hai? [Q79] Total layers kitni hain?

Section 7.3: Observed Machine UI Parameters & Cycle Timing (machine_parameters.md)

Record #15: Kelvin6K Observed Machine Parameters and Timing Procedural Content: Source: 3D_Concrete_Printing_Agent_Training_Dataset(2).pdf, sections 1 and 2. IMPORTANT: these are values READ OFF THE MACHINE SCREEN during one observation of one print job. They are a record of what the machine UI displays, not production standards. Kelvin6K_Printing_Data_Review_Problems_Solutions.pdf marks the 600 total layers as CONTEXT-SPECIFIC and the 1 minute 30 second layer time as OBSERVATION ONLY. These are NOT live values. The assistant must never answer a question about the CURRENT machine state from this file.

Record #16: Issue: Machine Parameter Snapshot from the Operator Screen Problem: Reference record of which parameters the machine UI displays, and the values observed during one print. Symptoms / Observations: Observed during printing on the machine operator screen. Machine status Running, Mode AUTO. Cause / Confidence: Not applicable - this is a reference record, not a fault. Solution / Corrective Action: Not applicable. Use this only to know which fields the machine reports. For the current values, read live machine telemetry, which is not yet connected to Kelvin6K. Safety / Important Notes: CONTEXT-SPECIFIC VALUES. The 600 total layers belongs to that specific print job and must not be treated as a universal wall layer count. The 150 percent feedrate override here is a snapshot reading, which is a different thing from the 150 percent recorded after a speed correction in the site incidents; see the conflicts file. None of these values describe the machine right now. Important Parameters: Machine status Running. Mode AUTO. Extruder 70 percent. Pump 24 percent. Fluid Pump 0 percent. Feedrate Override 150 percent. Current Feedrate 3600. Max Velocity 6000. Max Rapid 6000. Progress 20 percent. Completed Layers 115. Remaining Layers 485. Total layers indicated 600. Machine Run Time shown 00:10:42. Estimated Time shown 221.4 minutes. X position 1868.600. Y position 655.041. Z position 300.000. Velocity shown 3600. Verification Status: OBSERVED READING from one print, one moment. NOT a production standard. NOT live data. Source: PDF 2, Section 2. English Question Prompts: [Q80] What parameters does the machine display? [Q81] What was the extruder percentage? [Q82] How many layers were completed? [Q83] What does the machine UI show? [Q84] What is the max velocity? Hinglish Question Prompts: [Q85] Machine screen par kya kya dikhta hai? [Q86] Extruder kitne percent tha? [Q87] Kitni layers complete hui thi?

Record #17: Issue: Observed Layer Printing Time Problem: Reference record of how long one observed layer took to print. Symptoms / Observations: A 5-minute timer was used while observing one layer. At the end of the observation 3 minutes 30 seconds remained on the timer, so the elapsed printing time was 5:00 minus 3:30, which is 1:30. Cause / Confidence: Not applicable - this is an observation, not a fault. Solution / Corrective Action: Not applicable. Do not use this as a standard layer time or as the basis of an estimate without confirmation. Safety / Important Notes: OBSERVATION ONLY. This is a single observation of one layer and is not yet a standard layer time. The data review document marks it explicitly as observation only. Important Parameters: Timer duration 5 minutes. Time remaining 3 minutes 30 seconds. Elapsed time 1 minute 30 seconds. Observed layer time approximately 1 minute 30 seconds per layer. Verification Status: SINGLE OBSERVATION. NOT a confirmed standard. Source: PDF 2, Section 1. English Question Prompts: [Q88] How long does one layer take? [Q89] What is the layer time? [Q90] How many minutes per layer? Hinglish Question Prompts: [Q91] Ek layer kitni der mein print hoti hai? [Q92] Layer time kitna hai? [Q93] Ek layer mein kitna time lagta hai?

Section 7.4: Documented Printing Issues (PRINT Series) (printing_issues.md)

Record #18: Kelvin6K Documented Printing Issues (PRINT Series) Procedural Content: Source: Kelvin6K_Printing_Issues_and_Solutions_Dataset(2).pdf. IMPORTANT PROVENANCE NOTE: that document states it was generated from the Kelvin6K project ZIP - the operating manual, troubleshooting guide, print-monitor logic and alert rules. It is therefore project-derived, NOT operator-verified site knowledge. Every "Likely Cause" below is a hypothesis. It cannot be used to validate the system it was derived from. EXCLUSIONS applied to this file: - PRINT-008, PRINT-009 and PRINT-010 (high nozzle temperature, low nozzle temperature, high bed temperature) are EXCLUDED. This is a concrete printing machine with a pump, hopper and accelerator, and has no heated nozzle or heated bed. Nozzle temperature appears in none of the 50 real site incident records. These entries appear to originate from an FDM-shaped simulator. - PRINT-001, 002, 003, 004, 005, 006 and 018 are EXCLUDED as duplicates of docs/kelvin6k_manual.md and docs/kelvin6k_troubleshooting.md, which remain canonical for those topics.

Record #19: Issue: Low Material Flow Problem: Low material flow is reported; the print bead becomes narrow or material delivery is inconsistent. Symptoms / Observations: Agent reports low flow; print bead becomes narrow or material delivery is inconsistent. Cause / Confidence: Possible nozzle clog, low material supply, delivery-line issue, pump or seal issue, or air in the line. The root cause is not confirmed. Solution / Corrective Action: Inspect the material supply, the nozzle and the delivery line. Check the pump and seals if the issue continues. Use the nozzle-clog procedure when a blockage is suspected. Safety / Important Notes: Severity WARNING. Important Parameters: None specified in the source. Verification Status: DOCUMENTED (project-derived). Cause is LIKELY, not operator-confirmed. Source: PDF 3, PRINT-007. English Question Prompts: [Q94] Material flow is low. The bead is getting narrow. Why is material delivery inconsistent? Hinglish Question Prompts: [Q95] Material flow drop ho gaya hai. Material kam aa raha hai. Flow inconsistent hai.?

Record #20: Issue: Print Height Deviation or Layer Skipping Problem: Actual print height differs from the expected height, and layer pitch may be inconsistent. Symptoms / Observations: Actual print height differs from expected height; layer pitch may be inconsistent. Cause / Confidence: Layer skipping, or mechanical and printing inconsistency causing height drift. Not confirmed. Solution / Corrective Action: Pause and inspect the drive and gantry and the printed layer sequence. Check for mechanical problems and layer deposition consistency before continuing. Safety / Important Notes: Severity WARNING. See also the site-verified Z-height limit record, which is a different and machine-specific problem. Important Parameters: None specified in the source. Verification Status: DOCUMENTED (project-derived). Cause is LIKELY, not operator-confirmed. Source: PDF 3, PRINT-011. English Question Prompts: [Q96] The print height is wrong. Layers are being skipped. The height does not match the expected value.? Hinglish Question Prompts: [Q97] Print height expected se zyada ya kam ho rahi hai. Layer skip ho rahi hai. Height galat aa rahi hai.?

Record #21: Issue: Machine Error State Problem: Machine telemetry reports an error state. Symptoms / Observations: Machine telemetry reports an error state. Cause / Confidence: The machine or controller has reported an equipment error. Solution / Corrective Action: Pause the print and inspect the machine according to the reported error. Do not continue blindly while a critical machine error is active. Safety / Important Notes: Severity CRITICAL. For critical machine errors, pause or stop the relevant process and inspect the machine before continuing. Machine-specific commands are controlled procedures - AUTHORIZATION REQUIRED. Important Parameters: None generic. For specific recorded faults see the site-verified records for E08.5 and the servo drives. Verification Status: DOCUMENTED (project-derived). Source: PDF 3, PRINT-012. English Question Prompts: [Q98] The machine is showing an error. What do I do about a machine error? [Q99] There is a fault on the controller.? Hinglish Question Prompts: [Q100] Machine mein error aa raha hai. Controller error de raha hai. Error aane par kya karein?

Record #22: Issue: Pump Stopped Problem: The material pump is reported as stopped while printing is expected. Symptoms / Observations: Material pump is reported as stopped while printing is expected. Cause / Confidence: The pump has stopped or is not running. Solution / Corrective Action: Stop or hold the print as appropriate and check the pump, the power and control signal, the material path and the pump condition. Safety / Important Notes: Severity CRITICAL. See also the site-verified record where no ESP output voltage prevented the pump signal from reaching the pump. Important Parameters: None specified in the source. Verification Status: DOCUMENTED (project-derived). Source: PDF 3, PRINT-013. English Question Prompts: [Q101] The pump has stopped. Why is the pump not running? [Q102] The pump is reported stopped.? Hinglish Question Prompts: [Q103] Pump band ho gaya hai. Pump chal nahi raha. Pump kyun ruk gaya?

Record #23: Issue: Accelerator Stopped Problem: The accelerator is reported as stopped. Symptoms / Observations: Accelerator is reported as stopped. Cause / Confidence: The accelerator system has stopped or is not receiving the expected run signal. Solution / Corrective Action: Check accelerator operation, coupling and control signal. Resolve the issue before relying on normal material setting and curing behaviour. Safety / Important Notes: Severity CRITICAL. Important Parameters: None specified in the source. Verification Status: DOCUMENTED (project-derived). Source: PDF 3, PRINT-014. English Question Prompts: [Q104] The accelerator has stopped. Why is the accelerator not running? [Q105] Accelerator stopped during printing.? Hinglish Question Prompts: [Q106] Accelerator band ho gaya hai. Accelerator chal nahi raha. Accelerator kyun ruk gaya?

Record #24: Issue: Print Quality Too Low Problem: The overall measured print-quality score falls below the configured minimum. Symptoms / Observations: Overall measured print-quality score falls below the configured minimum. Cause / Confidence: One or more measurable print-quality indicators have degraded. Which one is not identified. Solution / Corrective Action: Inspect the printed wall and check the related measurements - layer width, layer detection, height, material flow and visible defects. Correct the underlying condition before continuing. Safety / Important Notes: Severity WARNING. Important Parameters: None specified in the source. Verification Status: DOCUMENTED (project-derived). Cause is LIKELY, not operator-confirmed. Source: PDF 3, PRINT-015. English Question Prompts: [Q107] The print quality score is low. Why is quality below the minimum? [Q108] Print quality has dropped.? Hinglish Question Prompts: [Q109] Print quality kharab aa rahi hai. Quality score kam hai. Print acha nahi aa raha.?

Record #25: Issue: Layer Missing While Printing Problem: The machine appears to be printing, but the vision system cannot confidently detect a layer. Symptoms / Observations: Machine appears to be printing, but the vision system cannot confidently detect a layer. Cause / Confidence: The layer is absent, obscured or poorly visible, or the vision measurement has insufficient confidence. Not confirmed. Solution / Corrective Action: Check the printed area and the camera view. Confirm lighting and framing and inspect whether material was actually deposited. Do NOT treat a low-confidence measurement as a confirmed physical failure without inspection. Safety / Important Notes: Severity WARNING. A low-confidence camera measurement must not be presented as a confirmed physical failure. Important Parameters: None specified in the source. Verification Status: DOCUMENTED (project-derived). Cause is LIKELY, not operator-confirmed. Source: PDF 3, PRINT-016. English Question Prompts: [Q110] The system says the layer is missing but the machine is printing. No layer detected. Vision cannot see the layer.? Hinglish Question Prompts: [Q111] Layer detect nahi ho rahi par machine chal rahi hai. Vision system layer nahi dekh raha.?

Record #26: Issue: Camera or Vision Error Problem: No usable camera frame or vision analysis is available. Symptoms / Observations: No usable camera frame or vision analysis is available. Cause / Confidence: The camera is disconnected, unavailable, or unable to provide a usable frame. Solution / Corrective Action: Check the camera connection, the power and USB connection, the camera selection and the camera view. Restore a valid frame before relying on vision-based printing status. Safety / Important Notes: Severity CRITICAL. This is a vision-system fault, which is distinct from the site-verified record about site camera image quality and network availability. Important Parameters: None specified in the source. Verification Status: DOCUMENTED (project-derived). Source: PDF 3, PRINT-017. English Question Prompts: [Q112] The camera is not working. No camera frame available. Vision system error.? Hinglish Question Prompts: [Q113] Camera feed nahi aa rahi. Vision system print detect nahi kar raha. Camera connect nahi hai.?

Record #27: Issue: Printing Status Not Reliable or Jumpy Problem: The agent alternates between printing and not printing, or between layer detected and missing, because of individual bad frames. Symptoms / Observations: Status flips between states on individual frames. Cause / Confidence: Single-frame noise, low-confidence measurements, or camera motion can cause unstable state decisions. Solution / Corrective Action: Use confidence thresholds and temporal debouncing. Inspect camera stability and the region of interest. The print monitor is designed to wait for consecutive agreeing frames rather than switching on one bad frame. Safety / Important Notes: Severity WARNING. Important Parameters: None numeric in the source. Verification Status: DOCUMENTED (project-derived). Source: PDF 3, PRINT-019. English Question Prompts: [Q114] The printing status keeps changing. Why is the status jumpy? [Q115] The state flips between printing and not printing.? Hinglish Question Prompts: [Q116] Printing status baar baar badal raha hai. Status stable nahi hai.?

Record #28: Issue: Printing Activity Not Detected by Camera Problem: The machine is physically moving and printing but the vision monitor reports no printing activity. Symptoms / Observations: Machine is physically moving or printing but the vision monitor reports no printing activity. Cause / Confidence: The motion calculation may be diluted by using the whole frame instead of the print region of interest, or the camera view and region of interest may be wrong. Solution / Corrective Action: Ensure motion is measured inside the configured region of interest around the deposition area. Check camera framing and region-of-interest configuration. Safety / Important Notes: Severity WARNING. Never tell the operator that a camera measurement is a confirmed machine state when the system is only inferring it from motion. Important Parameters: None numeric in the source. Verification Status: DOCUMENTED (project-derived). Cause is LIKELY, not operator-confirmed. Source: PDF 3, PRINT-020. English Question Prompts: [Q117] The camera does not detect printing but the machine is running. No printing activity detected. Motion is not being seen.? Hinglish Question Prompts: [Q118] Machine chal rahi hai par camera printing detect nahi kar raha. Motion detect nahi ho raha.?

Record #29: Issue: Printing Pipe Needs Cleaning Problem: Material flow becomes less reliable after extended operation, and a maintenance reminder appears. Symptoms / Observations: Material flow becomes less reliable after extended operation; maintenance reminder appears. Cause / Confidence: The printing pipe has accumulated material or residue and requires routine cleaning. Solution / Corrective Action: Clean the printing pipe every four hours of operation, as specified in the operating manual. Safety / Important Notes: Severity INFO. This is the PRINTING PIPE, cleaned every four HOURS. Do not confuse it with the ACCELERATOR HOLE, which is cleaned every five LAYERS with the big Allen key. They are different tasks with different intervals and different tools. Important Parameters: Pipe cleaning interval: every 4 hours of operation. Verification Status: DOCUMENTED (project-derived), consistent with the operating manual. Source: PDF 3, PRINT-021. English Question Prompts: [Q119] How often should the printing pipe be cleaned? [Q120] When was the pipe last cleaned? [Q121] What is the pipe cleaning interval? Hinglish Question Prompts: [Q122] Pipe ko last kab clean kiya tha? [Q123] Printing pipe kitne ghante mein clean karna hai?

Record #30: Issue: Accelerator Coupling Maintenance Problem: Routine accelerator coupling maintenance is due, or accelerator operation becomes questionable. Symptoms / Observations: Routine maintenance is due or accelerator operation becomes questionable. Cause / Confidence: Coupling wear or looseness, or an overdue inspection. Solution / Corrective Action: Inspect the accelerator coupling daily and correct any looseness or wear according to the machine maintenance procedure. Safety / Important Notes: Severity INFO. This is the accelerator COUPLING, inspected DAILY. Do not confuse it with the accelerator HOLE, cleaned every five LAYERS. Important Parameters: Accelerator coupling inspection interval: daily. Verification Status: DOCUMENTED (project-derived), consistent with the operating manual. Source: PDF 3, PRINT-022. English Question Prompts: [Q124] How often should the accelerator coupling be inspected? [Q125] Has the coupling been checked? [Q126] Accelerator coupling maintenance.? Hinglish Question Prompts: [Q127] Accelerator coupling kab check karna hai? [Q128] Coupling roz check karni hai?

Record #31: Issue: Pump Seal Maintenance Problem: Persistent flow problems occur even after checking material and nozzle. Symptoms / Observations: Persistent flow problems occur even after checking material and nozzle. Cause / Confidence: The pump seal may be worn or failing. Solution / Corrective Action: Inspect pump seals weekly and replace or service them according to the machine maintenance procedure if worn. Safety / Important Notes: Severity INFO. Distinct from cleaning the FRONT of the pump, which the site records specify after every two walls. Important Parameters: Pump seal inspection interval: weekly. Verification Status: DOCUMENTED (project-derived). Cause is LIKELY, not operator-confirmed. Source: PDF 3, PRINT-023. English Question Prompts: [Q129] How often should pump seals be inspected? [Q130] When were the pump seals last checked? [Q131] Pump seal maintenance.? Hinglish Question Prompts: [Q132] Pump seal kab check karna hai? [Q133] Pump ki seal weekly check karni hai?

Section 7.5: Machine Safety, Ambient Limits & Site PPE Rules (safety_rules.md)

Record #32: Kelvin6K Safety Rules to Preserve Procedural Content: Sources: Kelvin6K_Printing_Data_Review_Problems_Solutions.pdf section 6, and Kelvin6K_Printing_Issues_and_Solutions_Dataset(2).pdf section 5. These rules take priority over any other answer. The assistant is monitoring and advisory only and must never claim to have changed a machine setting.

Record #33: Issue: Machine and Print Safety Rules Problem: Certain printing conditions are safety issues, not merely quality issues, and acting on them in the wrong order can injure someone or damage the machine. Symptoms / Observations: A suspected nozzle clog, a critical machine error, severe delamination or layer shifting, or a machine-specific fault code. Cause / Confidence: Not applicable - these are standing rules. Solution / Corrective Action: The documented safety rules: Safety / Important Notes: AUTHORIZATION REQUIRED for machine-specific procedures, including the servo fault-clear parameter and the Z-height G-code corrections. Important Parameters: Nozzle-clog procedure from the operating manual: stop the pump, release system pressure, soak the nozzle in solvent for ten minutes, and clear it with the cleaning rod. Verification Status: DOCUMENTED SAFETY RULES. Source: PDF 1 section 6; PDF 3 section 5. English Question Prompts: [Q134] What are the safety rules? [Q135] Can I clear a clog while the pump is running? [Q136] What do I do first in a machine error? [Q137] Is it safe to keep printing after delamination? Hinglish Question Prompts: [Q138] Safety rules kya hain? [Q139] Pump chalu rehte hue clog saaf kar sakte hain? [Q140] Machine error mein pehle kya karna hai?

Record #34: Issue: Temperature Safety Rule - Applicability Unconfirmed Problem: The source documents include a safety rule about nozzle and bed temperature, but this machine may have neither. Symptoms / Observations: None observed. No temperature-related incident appears in any of the 50 real site records. Cause / Confidence: Not applicable. Solution / Corrective Action: The rule as written in the sources is: "High nozzle or bed temperature should be treated as a safety issue, not merely a print-quality issue." Confirm with the operator whether this machine has a heated nozzle or heated bed before acting on it. Safety / Important Notes: APPLICABILITY UNCONFIRMED. Kelvin6K is a concrete printing machine with a pump, hopper and accelerator. Nozzle or bed temperature appears in none of the 50 real site incident records. The related entries PRINT-008 (high nozzle temperature), PRINT-009 (low nozzle temperature) and PRINT-010 (high bed temperature) were excluded from the knowledge base for that reason. This rule is retained only because it appears in the source documents; it must be confirmed with the operator before being acted on. Important Parameters: None specified in the sources. Verification Status: APPLICABILITY UNCONFIRMED - pending operator confirmation. Likely not applicable to this machine. Source: PDF 1 section 6; PDF 3 section 5 and PRINT-008 to PRINT-010. English Question Prompts: [Q141] Does this machine have a heated nozzle? [Q142] Is nozzle temperature a safety issue? [Q143] What is the maximum bed temperature? Hinglish Question Prompts: [Q144] Is machine mein nozzle heater hai? [Q145] Nozzle temperature ka rule hai?

Record #35: Issue: Site Safety and PPE Problem: Site safety requirements were not consistently followed, and electrical hazards were left in place. Symptoms / Observations: Labourers not wearing masks or gloves; a loose or open wire present for days; walls damaged during lifting; robot movement near a tent and wires. Cause / Confidence: Safety requirements not enforced. Solution / Corrective Action: Enforce use of the provided PPE and site safety requirements. Install a proper plug or connector on any loose wire - no open wire should remain. Supervise lifting and crane movement and instruct labour on the correct lifting method. Move the tent before moving the machine, and take care of wires during movement. Safety / Important Notes: The source states safety equipment was provided. Open wiring is an electrical hazard and must not be left in place. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Source: PDF 4 records 37, 38, 43, 45. English Question Prompts: [Q146] What PPE is required on site? [Q147] Is the open wire safe? [Q148] How should walls be lifted safely? Hinglish Question Prompts: [Q149] Site par kya safety equipment pehnana hai? [Q150] Khula wire safe hai? [Q151] Wall safely kaise uthayein?

Section 7.6: Verified Field Site Incidents (WhatsApp Field Log) (site_incidents.md)

Record #36: Kelvin6K Site Incidents - Verified Printing Knowledge Procedural Content: Source: "WhatsApp Chat with Daily Printing Radiance Site.txt" (1,044 lines), via 3D_Printing_Issues_Solutions_AI_Agent_Dataset_No_Atom_V2(3).pdf. Dates 31 Aug - 6 Sep 2026. These are real site incidents with the actions actually taken. Media-only messages were not interpreted in the source, so some records are partial.

Record #37: Issue: Printing Delay - Y-Axis Error and Accelerator Setting Problem: Printing was delayed by a Y-axis error together with accelerator setting time. Symptoms / Observations: Print start was held up; shift time lost to troubleshooting. Cause / Confidence: Y-axis error and the time required for accelerator setup. Solution / Corrective Action: Resolve the Y-axis issue and complete accelerator setup before restarting. Plan the shift so troubleshooting does not unnecessarily stop printing. Safety / Important Notes: None specified in the source. Important Parameters: Target discussed: normally 2 walls per shift; small walls may allow more depending on size and print time. Recorded as "discussed", not confirmed as a formal target. Verification Status: SITE-VERIFIED. Solution Type: Corrective action. Source: PDF 4 record 1; Source Date 31 Aug 2026. English Question Prompts: [Q152] Why was printing delayed? [Q153] What is the target number of walls per shift? [Q154] How many walls should be printed in one shift? Hinglish Question Prompts: [Q155] Printing delay kyun hua? [Q156] Ek shift mein kitni wall print karni hai? [Q157] Y axis error aa raha hai.?

Record #38: Issue: Wall Count Not Updating Problem: The wall count was reported as not getting updated. Symptoms / Observations: Printing records did not reflect the walls actually printed. Cause / Confidence: The wall-count and record-update process was not being followed. Solution / Corrective Action: Check the wall-count and update process, and update the printing record when printing starts and when it completes. Safety / Important Notes: Keep wall numbers and print logs updated for traceability. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Process correction. Source: PDF 4 record 2; Source Date 31 Aug 2026. English Question Prompts: [Q158] Why is the wall count not updating? [Q159] How many walls have been printed? [Q160] The wall count is wrong.? Hinglish Question Prompts: [Q161] Wall count update nahi ho raha. Kitni wall print hui hai? [Q162] Wall count galat hai.?

Record #39: Issue: Dosing Pump Unavailable Problem: A dosing pump was required for the printing setup but was not available. Symptoms / Observations: The intended dosing arrangement could not be used. Cause / Confidence: Dosing pump not available on site. Solution / Corrective Action: Use the booster pump temporarily until the dosing pump is available. This is explicitly a temporary measure and NOT the permanent fix. The permanent fix is to obtain the dosing pump. Safety / Important Notes: A mechanical dosing pump was considered useful for continuous operation. Do not treat the booster pump as the final arrangement. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED as a TEMPORARY WORKAROUND. Solution Type: Temporary Workaround. Source: PDF 4 record 3; Source Date 31 Aug 2026. English Question Prompts: [Q163] What do I use if the dosing pump is not available? [Q164] Can the booster pump replace the dosing pump? Hinglish Question Prompts: [Q165] Dosing pump nahi hai to kya use karein? [Q166] Booster pump se kaam chalega?

Record #40: Issue: Mixer Chain Removed and Switch Broken Problem: The bigger mixer had a removed chain and a broken switch. Symptoms / Observations: Mixer not usable in that condition. Cause / Confidence: Chain removed; switch broken. Solution / Corrective Action: The chain was fixed. The switch was still to be fixed, and the mixer was to be tested afterward. Safety / Important Notes: Do not assume the repair is complete until the mixer has been tested. At the time of the record the switch repair was still outstanding. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED, REPAIR INCOMPLETE at time of record. Solution Type: Repair, not yet verified by test. Source: PDF 4 record 4; Source Date 31 Aug 2026. English Question Prompts: [Q167] What is wrong with the big mixer? [Q168] Is the mixer repaired? [Q169] The mixer switch is broken.? Hinglish Question Prompts: [Q170] Bada mixer kharab hai kya? [Q171] Mixer ka switch toot gaya hai. Mixer theek ho gaya?

Record #41: Issue: Material Not Coming Out of Hose Problem: Material stopped coming out of the hose. Symptoms / Observations: No material delivery at the hose outlet. Cause / Confidence: Not confirmed in the source. Hose condition and connections were the checks performed. Solution / Corrective Action: Open and inspect the hose; re-clamp where required and replace the hose if its condition is poor. Safety / Important Notes: Check hose connections before continuing. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Inspection and corrective action. Source: PDF 4 record 5; Source Date 31 Aug 2026. English Question Prompts: [Q172] Material is not coming out of the hose. Why has material delivery stopped? [Q173] The hose is not delivering material.? Hinglish Question Prompts: [Q174] Hose se material nahi aa raha. Material band ho gaya hai. Hose check karna hai.?

Record #42: Issue: Hose Repeatedly Choking or Clogging Problem: The hose repeatedly clogged. One print reached only 28 percent before the problem stopped it. Symptoms / Observations: Repeated choking at the hose; print unable to complete. Cause / Confidence: The source links choking to water and material leakage and to hardened material. Solution / Corrective Action: Clean the hose thoroughly. Replace the hose if one end keeps choking. Use another or a short hose temporarily. Safety / Important Notes: Using another or a short hose is a TEMPORARY measure; replacing the failing hose is the fix. This is the delivery hose, which is distinct from a nozzle clog. Important Parameters: One affected print reached only 28 percent completion. Verification Status: SITE-VERIFIED. Solution Type: Cleaning and replacement, with the substitute hose noted as TEMPORARY. Source: PDF 4 record 6; Source Date 1-3 Sep 2026. English Question Prompts: [Q175] Why does the hose keep choking? [Q176] The hose is clogged again. How do I fix a blocked hose? [Q177] Hose choking repeatedly.? Hinglish Question Prompts: [Q178] Hose baar baar choke ho raha hai. Hose block ho gaya hai. Hose clean kaise karein?

Record #43: Issue: Stone Stuck in Hose or Pump Problem: A stone became stuck in the hose during extrusion, and was later found in the hose even when pump inspection was clear. Symptoms / Observations: Blockage during extrusion; pump inspection alone did not reveal it. Cause / Confidence: Stone contamination in unscreened sand. Solution / Corrective Action: Clean the affected flow path, inspect the pump and the hose, and screen sand with a sieve or mesh. Safety / Important Notes: Do not start printing where stone contamination risk exists without filtration. A clear pump inspection does not rule out a stone in the hose. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Cleaning plus preventive filtration. Source: PDF 4 record 7; Source Date 1-3 Sep 2026. English Question Prompts: [Q179] There is a stone stuck in the hose. Why is the flow blocked by stones? [Q180] Stone in the pump.? Hinglish Question Prompts: [Q181] Hose mein patthar fas gaya hai. Pump mein stone aa gaya. Sand mein patthar hai.?

Record #44: Issue: Sand Screening or Filtration Missing Problem: Unscreened sand created a stone and blockage risk. Symptoms / Observations: Stones reaching the flow path. Cause / Confidence: Sand was not screened before use. Solution / Corrective Action: Use a sieve or mesh filtration. A mesh can be tied on top of the pump when fabrication is unnecessary. Safety / Important Notes: Instruction from the source: do not start printing without a sieve where required. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Preventive action. Source: PDF 4 record 8; Source Date 1 Sep 2026. English Question Prompts: [Q182] Do I need to sieve the sand? [Q183] How do I filter the sand? [Q184] Can I print without a sieve? Hinglish Question Prompts: [Q185] Sand ko sieve karna zaroori hai? [Q186] Mesh kaise lagayein? [Q187] Sieve ke bina printing kar sakte hain?

Record #45: Issue: Front Elevation Print Defect Problem: A front or elevation print was considered unacceptable because defects were visible. Symptoms / Observations: Visible defects on the elevation face. Cause / Confidence: Not confirmed in the source. Accelerator and speed were the settings adjusted. Solution / Corrective Action: Stop or correct the defective elevation print. Reduce accelerator and speed as needed and use fly-ash infill if instructed. Reprint when the result is unacceptable. Safety / Important Notes: Elevation quality has the highest priority. Defects in a single layer were stated to be unacceptable for elevation walls. Important Parameters: None numeric in this record. Related speed values appear in the printing-speed record. Verification Status: SITE-VERIFIED. Solution Type: Corrective action. Source: PDF 4 record 9; Source Date 2 Sep 2026. English Question Prompts: [Q188] The elevation print has defects. Should I reprint the elevation wall? [Q189] The front face looks bad.? Hinglish Question Prompts: [Q190] Elevation print mein defect hai. Front elevation kharab aa raha hai. Dobara print karna padega?

Record #46: Issue: Uneven Layer Thickness Problem: Layer thickness varied across the print. Symptoms / Observations: Thickness not consistent between layers. Cause / Confidence: Accelerator fluctuations were identified as a possible cause. The source records this as a possible cause, not a confirmed root cause. Solution / Corrective Action: Reduce accelerator by one number and clean the accelerator hole with the big Allen key. Safety / Important Notes: Do NOT use wire for accelerator-hole cleaning. Important Parameters: Accelerator reduction of one number. Tool: big Allen key. Verification Status: SITE-VERIFIED action; CAUSE POSSIBLE ONLY, not confirmed. Solution Type: Corrective action. Source: PDF 4 record 10; Source Date 2 Sep 2026. English Question Prompts: [Q191] Why is my layer thickness uneven? [Q192] The layers are not consistent. Layer thickness keeps changing. Uneven layer thickness.? Hinglish Question Prompts: [Q193] Layer thickness barabar nahi aa rahi. Layers uneven hain. Thickness badal rahi hai.?

Record #47: Issue: Printing Speed Too High Problem: A speed of 170 percent was reported as unacceptable for elevation printing. Symptoms / Observations: Elevation print quality unacceptable at that speed. Cause / Confidence: Speed too high for elevation-quality printing. Solution / Corrective Action: Reduce speed and accelerator ratio. A later update records 150 percent speed. Safety / Important Notes: Quality takes priority over speed for elevation walls. Important Parameters: 170 percent reported unacceptable for elevation. 150 percent recorded later. The source does not state that 150 percent is the formal maximum or a universal setting. Confirm the approved setting for the active print. See the conflicts file. Verification Status: SITE-VERIFIED that 170 percent was rejected. The approved standard value is NOT CONFIRMED. Source: PDF 4 record 11; Source Date 2 Sep 2026. English Question Prompts: [Q194] What speed should I use for elevation printing? [Q195] Is 170 percent too fast? [Q196] What is the correct feedrate? Hinglish Question Prompts: [Q197] Printing speed kitni rakhni hai? [Q198] 170 percent zyada hai? [Q199] Speed kam karni hai?

Record #48: Issue: Arc or Curved Print Problem: An arc or curve was observed in the print where it was not intended. Symptoms / Observations: The printed path deviated into a curve. Cause / Confidence: Not confirmed in the source. Print values, start lines and ground slope were the areas investigated. Solution / Corrective Action: Cross-check print values and start lines. Print thin layers to identify the deviation. Base-angle offsets may be needed to account for slope. Safety / Important Notes: None specified in the source. Important Parameters: One update recorded 3000 mm/min during this troubleshooting. Verification Status: SITE-VERIFIED investigation. Root cause NOT CONFIRMED. Source: PDF 4 record 12; Source Date 2 Sep 2026. English Question Prompts: [Q200] Why is my print curved? [Q201] There is an arc in the wall. The print is not straight.? Hinglish Question Prompts: [Q202] Print mein arc aa raha hai. Wall seedhi nahi aa rahi. Print curve ho raha hai.?

Record #49: Issue: Cracks in Printed Wall Problem: A large crack or excessive cracking was observed in a printed wall. Symptoms / Observations: Visible cracks in the printed wall. Cause / Confidence: Not confirmed in the source. Print and material conditions were the areas to inspect. Solution / Corrective Action: Fill the identified crack before shifting the wall, and inspect print and material conditions. Use fly ash where instructed. Safety / Important Notes: Cracks require inspection before proceeding. See also the unverified crack records in the visual-defects file, which describe crack appearance but carry no confirmed cause. Important Parameters: None specified in this record. Verification Status: SITE-VERIFIED action. Root cause NOT CONFIRMED. Source: PDF 4 record 13; Source Date 2 Sep 2026. English Question Prompts: [Q203] There is a crack in the wall. What do I do about cracks? [Q204] How do I fix a cracked wall? Hinglish Question Prompts: [Q205] Wall mein crack aa gaya hai. Crack kaise theek karein? [Q206] Deewar phat gayi hai.?

Record #50: Issue: Material Hardening During Pause Problem: Reused material with added water hardened between pauses despite flushing. Symptoms / Observations: Material set hard in the flow path during pauses. Cause / Confidence: Reused material with added water hardening while stationary. Solution / Corrective Action: Avoid unnecessary pauses. Keep the pump running periodically and clean the flow path when material hardens. Safety / Important Notes: Material condition is connected to choking risk. See also the older-batch record. Important Parameters: Keep the pump running every 5 minutes during relevant pauses. Verification Status: SITE-VERIFIED. Solution Type: Preventive action. Source: PDF 4 record 14; Source Date 2-6 Sep 2026. English Question Prompts: [Q207] The material hardened during the pause. How often should the pump run during a pause? [Q208] Material is setting in the line.? Hinglish Question Prompts: [Q209] Pause ke time material hard ho gaya. Pump kitni der mein chalana hai? [Q210] Material jam ho raha hai.?

Record #51: Issue: Power Cut Problem: A power cut delayed operations. Symptoms / Observations: Operations stopped for want of power. Cause / Confidence: Loss of mains power. Solution / Corrective Action: Turn on the DG, restore power, and repeat machine readiness checks before printing. Safety / Important Notes: Other preparation may still be required after power restoration. Do not assume the machine is ready simply because power is back. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Corrective action. Source: PDF 4 record 15; Source Date 2 Sep 2026. English Question Prompts: [Q211] What do I do after a power cut? [Q212] The power went out during printing. How do I restart after power failure? Hinglish Question Prompts: [Q213] Power cut ho gaya, kya karein? [Q214] Bijli chali gayi thi. DG chalana hai?

Record #52: Issue: Distributed Clock (DC) Startup Issue Problem: A startup issue was associated with Distributed Clock settings. Symptoms / Observations: The system would not start correctly. Cause / Confidence: DC inconsistency between master and slaves, or an unsupported or mismatched sync cycle. Solution / Corrective Action: Check DC consistency between master and slaves and verify a supported, identical sync cycle. Safety / Important Notes: MACHINE-SPECIFIC PROCEDURE - AUTHORIZATION REQUIRED. This is not a generic 3D-printer instruction. Important Parameters: SV660N sync values listed in the source: 250 microseconds, 500 microseconds, 1 millisecond, 2 milliseconds, 4 milliseconds. Verification Status: SITE-VERIFIED. Solution Type: Corrective action, machine-specific. Source: PDF 4 record 16; Source Date 2 Sep 2026. English Question Prompts: [Q215] What are the DC sync values? [Q216] Distributed clock startup problem. What sync cycle does the SV660N support? Hinglish Question Prompts: [Q217] DC settings ka issue hai. Sync cycle kitna rakhna hai? [Q218] Startup nahi ho raha.?

Record #53: Issue: J1 Ethernet Cable Fault Problem: A J1 failure was traced to a faulty Ethernet cable. Symptoms / Observations: J1 error reported. Cause / Confidence: Faulty Ethernet cable. Solution / Corrective Action: Temporarily connect using an available Ethernet cable, and route or replace the cable properly later. The temporary connection is explicitly not the permanent repair. Safety / Important Notes: The error was reported solved after the workaround, but proper routing or replacement was still outstanding. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED as a TEMPORARY WORKAROUND. Solution Type: Temporary Workaround. Source: PDF 4 record 17; Source Date 2 Sep 2026. English Question Prompts: [Q219] J1 error on the machine. The Ethernet cable is faulty. How do I fix a J1 fault? Hinglish Question Prompts: [Q220] J1 error aa raha hai. Ethernet cable kharab hai. J1 kaise theek karein?

Record #54: Issue: Servo Drives Not Turning ON Problem: The Z-axis and X-axis servo drives were not turning ON. Symptoms / Observations: Drives failed to power up. Cause / Confidence: Two MCV wires were found disconnected. Solution / Corrective Action: Check the wiring. Two MCV wires were found disconnected, were reconnected properly, and the system was then restarted. Safety / Important Notes: Also check the drive power cable and the line neutral during diagnosis. MACHINE-SPECIFIC - AUTHORIZATION REQUIRED for electrical work. Important Parameters: Two MCV wires affected. Axes involved: Z and X. Verification Status: SITE-VERIFIED, cause confirmed as disconnected wiring. Solution Type: Permanent Fix. Source: PDF 4 record 18; Source Date 5 Sep 2026. English Question Prompts: [Q221] The servo drives are not turning on. Z axis drive will not power up. Why are the servo drives off? Hinglish Question Prompts: [Q222] Servo drive ON nahi ho raha. Z axis drive chalu nahi ho raha. Wiring check karni hai?

Record #55: Issue: Z-Axis Servo Fault E08.5 Problem: The Z-axis servo drive showed fault code E08.5. Symptoms / Observations: EtherCAT reconnection and system restarts did not clear the fault. Cause / Confidence: Servo drive fault condition E08.5. The underlying physical cause is not recorded in the source. Solution / Corrective Action: In the J2 servo drive, set parameter H0D20 greater than 1 (the fault-clear parameter) and verify. The source reports the fault cleared this way. Safety / Important Notes: MACHINE-SPECIFIC PROCEDURE - AUTHORIZATION REQUIRED. This is not a generic 3D-printer instruction. Identify the exact machine condition and follow the authorized procedure. For critical machine errors, pause the print and inspect before continuing. Important Parameters: Fault code E08.5. Drive J2. Parameter H0D20, value greater than 1. Verification Status: SITE-VERIFIED - the source reports the fault cleared this way. Solution Type: Corrective action, machine-specific. Source: PDF 4 record 19; Source Date 5 Sep 2026. English Question Prompts: [Q223] How do I clear E08.5? [Q224] The Z-axis servo shows E08.5. What is the fault-clear parameter for the servo drive? [Q225] E08.5 servo fault.? Hinglish Question Prompts: [Q226] Z axis servo mein E08.5 error aa raha hai. Servo fault kaise clear karein? [Q227] E08.5 ka solution kya hai?

Record #56: Issue: Robot Incorrect Position or Alignment Problem: The robot was not placed in the correct position and needed hydraulic movement. Symptoms / Observations: Robot position wrong for the intended print. Cause / Confidence: Robot positioned incorrectly. Solution / Corrective Action: Move the robot carefully using hydraulic movement, then perform alignment and print calibration. Safety / Important Notes: Move carefully. See also the tent-collision record before moving the machine. Important Parameters: Cleaning, alignment and restart should fit within the one-hour window specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Corrective action. Source: PDF 4 record 20; Source Date 5-6 Sep 2026. English Question Prompts: [Q228] The robot is in the wrong position. How do I align the robot? [Q229] Robot needs to be moved.? Hinglish Question Prompts: [Q230] Robot sahi position mein nahi hai. Robot ko move karna hai. Alignment karni hai.?

Record #57: Issue: E-Axis Mixer Not Turning ON Problem: The E-axis servo was running but the E-axis mixer did not turn ON. Symptoms / Observations: Servo running, mixer not starting. Cause / Confidence: A disconnected USB cable between the ESP box and the ESP. Solution / Corrective Action: Inspect the ESP connection. Reconnect the disconnected USB cable from the ESP box to the ESP and restart. Safety / Important Notes: Printing resumed after reconnection. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED, cause confirmed. Solution Type: Permanent Fix. Source: PDF 4 record 21; Source Date 5 Sep 2026. English Question Prompts: [Q231] The E-axis mixer is not turning on. The mixer will not start but the servo runs. ESP connection problem.? Hinglish Question Prompts: [Q232] E axis mixer ON nahi ho raha. Servo chal raha hai par mixer nahi. ESP ka cable check karna hai.?

Record #58: Issue: E-Mixer and Pump Not Working Problem: There was no ESP output voltage and the pump signal was not reaching the pump. Symptoms / Observations: Neither the E-mixer nor the pump operated. Cause / Confidence: ESP data cable fault, and a problem in the long pump wire. Solution / Corrective Action: Check ESP voltage. Replace the data cable supplying the ESP and recheck. Inspect the long pump wire, cut a small portion from both ends and reconnect properly. Safety / Important Notes: Both the E-mixer and the pump were reported working afterward. MACHINE-SPECIFIC - AUTHORIZATION REQUIRED for electrical work. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED, reported working afterward. Solution Type: Permanent Fix. Source: PDF 4 record 22; Source Date 5-6 Sep 2026. English Question Prompts: [Q233] The pump and mixer are both not working. No ESP output voltage. The pump signal is not reaching the pump.? Hinglish Question Prompts: [Q234] Pump aur mixer dono band hain. ESP se voltage nahi aa raha. Pump ko signal nahi ja raha.?

Record #59: Issue: Micro USB Type-A Cable Failure Problem: An old Micro USB Type-A cable was not working. Symptoms / Observations: Connection failure through that cable. Cause / Confidence: Cable failure through age or wear. Solution / Corrective Action: Replace the cable and keep a spare available. Safety / Important Notes: Replacement allowed operation to continue. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Permanent Fix. Source: PDF 4 record 23; Source Date 5 Sep 2026. English Question Prompts: [Q235] The USB cable is not working. Do we have a spare USB cable? [Q236] Micro USB cable failure.? Hinglish Question Prompts: [Q237] USB cable kaam nahi kar raha. Naya cable lagana hai. Spare cable hai?

Record #60: Issue: VFD Wire Issue Problem: A VFD wire connection problem was found. Symptoms / Observations: Connection fault at the VFD wiring. Cause / Confidence: Wire connection problem. Solution / Corrective Action: Check the wire, trim the affected wire if required, reconnect and test. Safety / Important Notes: Reported working after reconnection. MACHINE-SPECIFIC - AUTHORIZATION REQUIRED for electrical work. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Permanent Fix. Source: PDF 4 record 24; Source Date 5 Sep 2026. English Question Prompts: [Q238] There is a VFD wire problem. The VFD connection is faulty. How do I fix the VFD wiring? Hinglish Question Prompts: [Q239] VFD ki wire mein problem hai. VFD connection check karna hai.?

Record #61: Issue: Z-Height Limit - Print Stopped Before Completion Problem: A print stopped with approximately 80 mm remaining because the Z-height limit was not set correctly. This was linked to the removal of the Z motor. Symptoms / Observations: Print halted before the wall was finished. Cause / Confidence: Z-height or limit not set correctly, linked to removal of the Z motor. One message in the source identifies this as a Z-offset issue rather than necessarily a G-code error. Solution / Corrective Action: Check the Z-height during calibration. Manually correct the relevant file or J2 height and resume. The source records G92.1 followed by G92 Z0 at the current position, or setting a suitable Z value such as G92 Z1515. See the conflicts file - the source records more than one approach. Safety / Important Notes: MACHINE-SPECIFIC PROCEDURE - AUTHORIZATION REQUIRED. These are controlled procedures, not generic troubleshooting instructions. The source itself is not consistent about whether this is a Z-offset or a G-code problem. Important Parameters: Approximately 80 mm remaining at the stop. G-code recorded: G92.1, then G92 Z0. Alternative recorded: G92 Z1515. Verification Status: SITE-VERIFIED action; the source records conflicting characterisations of the cause. Solution Type: Corrective action, machine-specific. Source: PDF 4 record 25; Source Date 5-6 Sep 2026. English Question Prompts: [Q240] The print stopped before finishing. Why did the print stop with 80 mm left? [Q241] How do I fix the Z height? [Q242] Z offset problem.? Hinglish Question Prompts: [Q243] Print pura hone se pehle ruk gaya. Z height galat hai. 80 mm bacha tha aur ruk gaya.?

Record #62: Issue: U-Joint Not Joining Problem: U-joints between walls did not join because of a layer-size difference. Symptoms / Observations: The joint between walls failed to close. Cause / Confidence: Layer-size difference at the joint; 40 mm overlapped the adjacent layer. Solution / Corrective Action: Check the layer thickness at the U-joint and adjust the model dimension. 40 mm was changed to 50 mm because 40 mm overlapped the adjacent layer. Another message suggested plus 15 mm, subject to checking thickness and speed. Safety / Important Notes: Verify actual joint fit before repeating. Important Parameters: 40 mm changed to 50 mm. Plus 15 mm suggested as an alternative, subject to checking thickness and speed. See the conflicts file - layer thickness values differ between contexts. Verification Status: SITE-VERIFIED. Solution Type: Corrective action. Source: PDF 4 record 26; Source Date 5 Sep 2026. English Question Prompts: [Q244] The U-joint is not joining. What layer thickness for the U-joint? [Q245] The joint between walls has a gap.? Hinglish Question Prompts: [Q246] U-joint join nahi ho raha. Joint mein gap hai. U-joint ki thickness kitni rakhein?

Record #63: Issue: Wrong G-Code or Wrong File Problem: An incorrectly sliced or selected file created confusion about which wall was being printed. Symptoms / Observations: Uncertainty about the wall being printed. Cause / Confidence: Incorrect slicing or file selection. Solution / Corrective Action: Verify the G-code, the file name and the panel image before printing. Re-slice or re-upload incorrect files and mark the wrong G-code. Safety / Important Notes: None specified in the source beyond verification before printing. Important Parameters: Create a verified sequence of at least 20 prints. Verification Status: SITE-VERIFIED. Solution Type: Process correction. Source: PDF 4 record 27; Source Date 6 Sep 2026. English Question Prompts: [Q247] Which wall is being printed? [Q248] The G-code is wrong. How do I verify the file before printing? [Q249] Wrong file selected.? Hinglish Question Prompts: [Q250] Galat G-code select ho gaya. Kaunsi wall print ho rahi hai? [Q251] File verify kaise karein?

Record #64: Issue: File Naming Mismatch Problem: A file name did not clearly match the actual wall, causing uncertainty. Symptoms / Observations: Operators unsure whether the correct wall was loaded. Cause / Confidence: File naming not aligned with the actual wall. Solution / Corrective Action: Use the actual file name. Confirm the wall number, G-code and image. If only the name is wrong but the wall is correct and not yet printed, the source says not to stop solely for naming. Safety / Important Notes: Communicate the next wall clearly to the next shift. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Process correction. Source: PDF 4 record 28; Source Date 6 Sep 2026. English Question Prompts: [Q252] The file name does not match the wall. Should I stop printing for a naming mistake? [Q253] File naming mismatch.? Hinglish Question Prompts: [Q254] File ka naam wall se match nahi kar raha. Naam galat hai to rokna chahiye?

Record #65: Issue: Layer Thickness Too High Problem: Layers were reported as too thick. Symptoms / Observations: Thick layers in the printed wall. Cause / Confidence: Layer thickness setting too high for the print. Solution / Corrective Action: Reduce the layer thickness. In that discussion the operator reported 40 mm layer thickness and 120 mm distance. Use the verified setting for the print. Safety / Important Notes: Fill the bottom three layers with fly ash and add fly ash around layers where instructed. Note that another record specifies fly ash up to five layers; see the conflicts file. Important Parameters: 40 mm layer thickness reported, 120 mm distance. See the conflicts file - 40 mm appears as both too thick here and as the value replaced by 50 mm at the U-joint. Verification Status: SITE-VERIFIED. Solution Type: Corrective action. Source: PDF 4 record 29; Source Date 6 Sep 2026. English Question Prompts: [Q255] The layers are too thick. What layer thickness should I use? [Q256] How do I reduce layer thickness? Hinglish Question Prompts: [Q257] Layers bahut thick hain. Layer thickness kam karni hai. Kitni thickness rakhein?

Record #66: Issue: Bottom Sand Compaction and Base Instability Problem: A defect was attributed to insufficient attention during bottom sand compaction. A fallen-wall cause was later investigated. Symptoms / Observations: Base-related defect; a wall subsequently fell. Cause / Confidence: Attributed initially to insufficient bottom sand compaction. A later assessment suggested wet layers were more likely than sand compression in that case. See the conflicts file. Solution / Corrective Action: Prepare the base correctly - level the sand surface, place polythene, then print. Fill fly ash up to five layers inside and outside as instructed. Safety / Important Notes: For a fallen wall, check CCTV and layer condition. Important Parameters: Fly ash up to five layers inside and outside. Note that another record specifies bottom three layers; see the conflicts file. Verification Status: SITE-VERIFIED action; CAUSE DISPUTED between two records. Solution Type: Preventive action. Source: PDF 4 record 30; Source Date 2 and 6 Sep 2026. English Question Prompts: [Q258] How do I prepare the base before printing? [Q259] Why did the wall fall? [Q260] Bottom sand compaction. Do I need polythene? Hinglish Question Prompts: [Q261] Base kaise taiyaar karein? [Q262] Wall kyun gir gayi? [Q263] Polythene lagana hai? [Q264] Sand compaction kaise karein?

Record #67: Issue: Wet Layers and Wall Falling Problem: A wall fell and the team investigated whether sand compression or wet layers caused it. Symptoms / Observations: A printed wall collapsed. Cause / Confidence: The source says the observed pattern looked more consistent with wet layers than with sand compression. This is an assessment, not a confirmed root cause. Solution / Corrective Action: Check CCTV and layer condition. Ensure layers are sufficiently dry before proceeding. Safety / Important Notes: Investigate wall instability with CCTV and physical inspection. See also the bottom sand compaction record, which attributes a related defect differently. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED observation; CAUSE POSSIBLE ONLY, and disputed against the sand-compaction record. Source: PDF 4 record 31; Source Date 6 Sep 2026. English Question Prompts: [Q265] Why did the wall fall down? [Q266] Were the layers too wet? [Q267] What causes a wall to collapse? Hinglish Question Prompts: [Q268] Wall gir gayi, kyun? [Q269] Layers geeli thi kya? [Q270] Wall kyun collapse hui?

Record #68: Issue: Rain Interruption Problem: Rain forced setup closure and stopped printing. Symptoms / Observations: Printing halted for weather. Cause / Confidence: Rain. Solution / Corrective Action: After rain, reopen and prepare the setup, perform alignment, and restart only when ready. Safety / Important Notes: Alignment was performed before restart. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Corrective action. Source: PDF 4 record 32; Source Date 3 Sep 2026. English Question Prompts: [Q271] What do I do when it rains? [Q272] Printing stopped because of rain. Can I print after rain? Hinglish Question Prompts: [Q273] Baarish ho gayi, kya karein? [Q274] Rain ke baad printing kab shuru karein?

Record #69: Issue: Hopper Motor Heating Problem: The hopper motor was reported to be heating at night. Symptoms / Observations: Motor running hot. Cause / Confidence: Not confirmed in the source. Solution / Corrective Action: Keep checking the motor. During daytime, spray the E-motor with fresh water as instructed. Safety / Important Notes: Monitor heating. The source specifies fresh tank water. Important Parameters: Use fresh tank water in a spray bottle. Verification Status: SITE-VERIFIED action. Root cause NOT CONFIRMED. Solution Type: Monitoring and mitigation. Source: PDF 4 record 33; Source Date 3 and 6 Sep 2026. English Question Prompts: [Q275] The hopper motor is getting hot. What do I do about motor heating? [Q276] Can I spray water on the motor? Hinglish Question Prompts: [Q277] Hopper motor garam ho raha hai. Motor par paani spray karna hai? [Q278] Motor heating ka kya karein?

Record #70: Issue: Accelerator Hole Blockage Problem: The accelerator hole or pipe can block and affect print quality. Symptoms / Observations: Print quality affected; related to uneven layer thickness. Cause / Confidence: Accumulated material blocking the accelerator hole or pipe. Solution / Corrective Action: Clean the accelerator hole every five layers with the big Allen key or an appropriate rod. If the machine must be stopped, stop in the column area and keep the stop under five seconds as instructed. Safety / Important Notes: Do NOT use wire for this cleaning. Important Parameters: Cleaning interval every 5 layers. Maximum stop duration 5 seconds. Stop location: column area. Tool: big Allen key or appropriate rod. Verification Status: SITE-VERIFIED. Solution Type: Preventive and corrective action. Source: PDF 4 record 34; Source Date 2 and 6 Sep 2026. English Question Prompts: [Q279] How often should the accelerator hole be cleaned? [Q280] What tool cleans the accelerator hole? [Q281] The accelerator hole is blocked. Accelerator hole blockage.? Hinglish Question Prompts: [Q282] Accelerator hole kab clean karna hai? [Q283] Accelerator hole block ho gaya hai. Kitni layers ke baad clean karna hai?

Record #71: Issue: Pump Exposed to Direct Sun Problem: The pump was at risk of heating when exposed to sun. Symptoms / Observations: Risk of pump overheating. Cause / Confidence: Direct sun exposure. Solution / Corrective Action: Keep the pump under shade and avoid unnecessary pauses. Run the pump periodically to prevent choking. Safety / Important Notes: The source explicitly says to keep the pump under shade. Important Parameters: None specified in this record. The related material-hardening record specifies running the pump every 5 minutes during pauses. Verification Status: SITE-VERIFIED. Solution Type: Preventive action. Source: PDF 4 record 35; Source Date 6 Sep 2026. English Question Prompts: [Q284] Should the pump be in the shade? [Q285] The pump is getting hot in the sun. How do I stop the pump overheating? Hinglish Question Prompts: [Q286] Pump ko dhoop se bachana hai? [Q287] Pump garam ho raha hai. Pump shade mein rakhna hai?

Record #72: Issue: Limited Printing Space Problem: Random or side printing blocked future positions and available space became limited. Symptoms / Observations: No room left for planned prints. Cause / Confidence: Print locations chosen without planning. Solution / Corrective Action: Plan print locations. Print parallel where possible. Avoid side prints that block future positions. Clear space before starting. Safety / Important Notes: Planning multiple reception walls in one area was suggested to reduce downtime. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Preventive planning. Source: PDF 4 record 36; Source Date 1-3 Sep 2026. English Question Prompts: [Q288] There is no space left to print. How should I plan print positions? [Q289] Printing space is limited.? Hinglish Question Prompts: [Q290] Printing ke liye jagah nahi bachi. Print position kaise plan karein? [Q291] Space clear karna hai.?

Record #73: Issue: Tent Collision and Wire Risk Problem: The robot could collide with the tent while moving, and wires could also be damaged. Symptoms / Observations: Collision and wire-damage risk during machine movement. Cause / Confidence: Tent in the machine's movement path; exposed wires. Solution / Corrective Action: Move the tent first, then the machine. Be careful with wires during movement. Safety / Important Notes: Do not move the machine blindly when the tent is in the collision path. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Preventive action. Source: PDF 4 record 37; Source Date 2 Sep 2026. English Question Prompts: [Q292] Can the robot hit the tent? [Q293] What do I move first, the tent or the machine? [Q294] Wire damage during movement.? Hinglish Question Prompts: [Q295] Robot tent se takra sakta hai? [Q296] Pehle tent hatana hai? [Q297] Wire ka dhyan rakhna hai.?

Record #74: Issue: Wall Damage During Lifting Problem: A wall was picked or lifted from the wrong side and risked damage. Symptoms / Observations: Damage risk during lifting. Cause / Confidence: Incorrect lifting method. Solution / Corrective Action: Tell the labour the correct lifting method and supervise lifting and crane movement. Safety / Important Notes: The source specifically warns that careless lifting can damage walls. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Preventive action. Source: PDF 4 record 38; Source Date 4 Sep 2026. English Question Prompts: [Q298] How should the wall be lifted? [Q299] The wall was damaged during lifting. Which side do we lift from? Hinglish Question Prompts: [Q300] Wall ko kaise uthana hai? [Q301] Lifting mein wall damage ho gayi. Kis side se uthana hai?

Record #75: Issue: Curing Water Hose Unavailable Problem: A water hose was needed for curing but the site had no stock; buckets took more time. Symptoms / Observations: Curing slowed by lack of a hose. Cause / Confidence: No hose in stock. Solution / Corrective Action: Arrange a water hose in advance, or obtain one from the site team when available. Safety / Important Notes: None specified in the source. Important Parameters: Printed walls were instructed to be cured three times per day. Verification Status: SITE-VERIFIED. Solution Type: Preventive planning. Source: PDF 4 record 39; Source Date 1-3 Sep 2026. English Question Prompts: [Q302] How often should walls be cured? [Q303] We have no water hose for curing. Curing schedule.? Hinglish Question Prompts: [Q304] Wall ko din mein kitni baar curing karni hai? [Q305] Curing ke liye hose nahi hai. Paani kaise dalein?

Record #76: Issue: Insufficient Labour Problem: Printing was delayed due to a lack of labour for preparation and space clearing. Symptoms / Observations: Preparation not completed in time. Cause / Confidence: Insufficient labour allocated. Solution / Corrective Action: Coordinate labour before starting. Maintain the required labour for printing, and use site labour for heavy unrelated work where possible. Safety / Important Notes: The source repeatedly emphasises printing as the primary job. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Process correction. Source: PDF 4 record 40; Source Date 31 Aug - 3 Sep 2026. English Question Prompts: [Q306] Printing is delayed because of labour. How much labour is needed? [Q307] Not enough workers.? Hinglish Question Prompts: [Q308] Labour kam hai isliye delay ho raha hai. Kitne log chahiye? [Q309] Labour arrange karna hai.?

Record #77: Issue: Crane Delay Problem: A crane was delayed and a call was not being answered. Symptoms / Observations: Crane not available when needed. Cause / Confidence: Vendor or coordination delay. Solution / Corrective Action: Follow up with the responsible person or vendor, and continue other productive preparation in parallel. Safety / Important Notes: The goal is to minimise printing downtime. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Process correction. Source: PDF 4 record 41; Source Date 5 Sep 2026. English Question Prompts: [Q310] The crane has not arrived. What do we do while waiting for the crane? Hinglish Question Prompts: [Q311] Crane nahi aaya hai. Crane ka wait karte hue kya karein?

Record #78: Issue: Older Batch Getting Hard Problem: An older material batch was reported as getting hard. Symptoms / Observations: Material hardening in the older batch. Cause / Confidence: Batch age and condition. Solution / Corrective Action: Identify batch age and condition, avoid unnecessary pauses, and clean the affected flow path if material hardens. Safety / Important Notes: Material condition is connected to choking risk. See also the material-hardening-during-pause record. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Corrective action. Source: PDF 4 record 42; Source Date 5 Sep 2026. English Question Prompts: [Q312] The old batch is getting hard. Can I use older material? [Q313] Material is hardening.? Hinglish Question Prompts: [Q314] Purana batch hard ho gaya hai. Purana material use kar sakte hain? [Q315] Material jam ho raha hai.?

Record #79: Issue: Loose or Open Wire Problem: A loose or open wire had reportedly been present for days. Symptoms / Observations: Exposed wiring left in place. Cause / Confidence: Wiring not properly terminated. Solution / Corrective Action: Install a proper plug or connector and fix the wire. Do not leave open wiring. Safety / Important Notes: Instruction from the source: no open wire like this should remain. This is an electrical safety issue. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Permanent Fix required. Source: PDF 4 record 43; Source Date 6 Sep 2026. English Question Prompts: [Q316] There is an open wire on site. Loose wire hanging. Is exposed wiring safe? Hinglish Question Prompts: [Q317] Wire khula pada hai. Loose wire hai, theek karna hai. Wire safe nahi hai.?

Record #80: Issue: Camera Quality and Network Issue Problem: The camera initially had poor image quality and the site had no local network for remote viewing. Symptoms / Observations: Poor image; no remote viewing available. Cause / Confidence: Unsuitable camera and absence of a local network. Solution / Corrective Action: Fix or install a suitable camera and power. If there is no local network, record locally and use the site hotspot for viewing when needed. Safety / Important Notes: The camera is intended for timelapse and remote monitoring. This is distinct from a vision-system camera error inside the monitoring software. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Corrective action. Source: PDF 4 record 44; Source Date 2-3 Sep 2026. English Question Prompts: [Q318] The camera image is poor. There is no network for the camera. How do I view the camera remotely? Hinglish Question Prompts: [Q319] Camera ki quality kharab hai. Network nahi hai camera ke liye. Camera remote se kaise dekhein?

Record #81: Issue: Safety Equipment Not Worn Problem: Labourers were not consistently wearing masks and gloves. Symptoms / Observations: PPE not in use despite being provided. Cause / Confidence: PPE use not enforced. Solution / Corrective Action: Enforce use of the provided PPE and site safety requirements. Safety / Important Notes: The source says safety equipment was provided. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Process enforcement. Source: PDF 4 record 45; Source Date 6 Sep 2026. English Question Prompts: [Q320] Are workers wearing PPE? [Q321] Masks and gloves are not being worn. What safety equipment is required? Hinglish Question Prompts: [Q322] Mask aur gloves nahi pehen rahe hain. PPE zaroori hai? [Q323] Safety equipment kahan hai?

Record #82: Issue: Fly Ash and Empty Bag Shortage Problem: Fly ash stock became low and empty bags were needed. Symptoms / Observations: Consumables running out. Cause / Confidence: Stock not replenished in advance. Solution / Corrective Action: Arrange fly ash and empty bags before stock runs out. Safety / Important Notes: Fly ash is used for bottom-layer filling and defect correction. Important Parameters: None specified in this record. Fly ash quantities appear in the layer-thickness and base-preparation records, where they conflict; see the conflicts file. Verification Status: SITE-VERIFIED. Solution Type: Preventive planning. Source: PDF 4 record 46; Source Date 2-3 Sep 2026. English Question Prompts: [Q324] Is there enough fly ash? [Q325] We need empty bags. Fly ash stock is low.? Hinglish Question Prompts: [Q326] Fly ash khatam ho raha hai. Empty bags chahiye. Fly ash ka stock kitna hai?

Record #83: Issue: Reinforcement Wire Stock Exhausted Problem: Reinforcement wire stock ran out. Symptoms / Observations: No reinforcement wire available. Cause / Confidence: Stock not tracked or replenished. Solution / Corrective Action: Purchase and bring reinforcement wire to site before it is needed. Safety / Important Notes: Track stock proactively. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Preventive planning. Source: PDF 4 record 47; Source Date 5-6 Sep 2026. English Question Prompts: [Q327] Do we have reinforcement wire? [Q328] The reinforcement wire has run out.? Hinglish Question Prompts: [Q329] Reinforcement wire khatam ho gaya. Wire ka stock hai?

Record #84: Issue: Material or Sand Quantity Low Problem: The team had to request more sand or material when the quantity was insufficient. Symptoms / Observations: Material shortage during operation. Cause / Confidence: Quantity not communicated or planned in advance. Solution / Corrective Action: Use the specified sand stock and request a reload from the site team when material is low. Safety / Important Notes: Requirements should be communicated in advance. Important Parameters: None specified in the source. Verification Status: SITE-VERIFIED. Solution Type: Preventive planning. Source: PDF 4 record 48; Source Date 3 Sep 2026. English Question Prompts: [Q330] We are running out of sand. How much material is left? [Q331] Request more material.? Hinglish Question Prompts: [Q332] Sand kam pad raha hai. Material khatam ho raha hai. Aur material mangwana hai.?

Record #85: Issue: Cleaning and Alignment Restart Taking Too Long Problem: Repeated reminders said that cleaning, alignment and restart were delaying the next print. Symptoms / Observations: Long gap between finishing one wall and starting the next. Cause / Confidence: Cleaning and alignment not completed within the expected window. Solution / Corrective Action: After each wall, clean the hopper. After every two walls, clean the front of the pump. Complete alignment and restart within one hour. Safety / Important Notes: One hour is repeatedly stated as the maximum restart window. Important Parameters: Hopper cleaning after each wall. Pump-front cleaning after every 2 walls. Alignment and restart within 1 hour maximum. Verification Status: SITE-VERIFIED. Solution Type: Process correction. Source: PDF 4 record 49; Source Date 6 Sep 2026. English Question Prompts: [Q333] How long should the restart take? [Q334] When do I clean the hopper? [Q335] When do I clean the front of the pump? [Q336] What is the restart window? Hinglish Question Prompts: [Q337] Restart kitni der mein karna hai? [Q338] Hopper kab clean karna hai? [Q339] Pump ka front kab clean karna hai?

Record #86: Issue: Poor Shift Handover or Unclear Next Wall Problem: Operators repeatedly asked which wall or file should be printed next. Symptoms / Observations: Uncertainty at shift change about the next print. Cause / Confidence: Print sequence and handover not clearly communicated. Solution / Corrective Action: Maintain a clear print sequence. Verify the wall number, G-code, file name and image, and communicate the next wall to the incoming shift. Safety / Important Notes: None specified in the source. Important Parameters: A sequence of at least 20 prints was requested. Verification Status: SITE-VERIFIED. Solution Type: Process correction. Source: PDF 4 record 50; Source Date 2-6 Sep 2026. English Question Prompts: [Q340] Which wall is next? [Q341] What should I print next? [Q342] The next wall is not clear. Shift handover.? Hinglish Question Prompts: [Q343] Agli wall kaunsi hai? [Q344] Ab kya print karna hai? [Q345] Next wall ka pata nahi hai.?

Section 7.7: Observed Visual Defects & Void Classifications (visual_defects.md)

Record #87: Kelvin6K Visual Defect Observations - UNVERIFIED Procedural Content: Source: 3D_Concrete_Printing_Agent_Training_Dataset(2).pdf, sections 3 to 6. IMPORTANT: every record in this file is UNVERIFIED. The source states for each one that the confirmed root cause is "Not confirmed from the images alone" and that the confirmed solution is "To be verified with the operator/technician". Section 8 of that document states that information marked as possible causes is based on visual observation and must NOT be treated as a confirmed diagnosis. The assistant must never present any cause in this file as confirmed. Correct wording is "Possible causes include ... The root cause is not confirmed yet."

Record #88: Issue: Layer Gap or Void Problem: A local gap or void is visible in the printed concrete layers. Symptoms / Observations: A section of the wall shows incomplete filling or bonding between printed material. Cause / Confidence: Possible causes include inconsistent material flow, temporary extrusion interruption, extrusion or pump setting mismatch, material consistency and workability variation, or poor inter-layer bonding. The root cause is NOT confirmed. The source states it was not confirmed from the images alone and must be verified with the operator or technician. Solution / Corrective Action: Troubleshooting to verify - check material flow, extruder and pump settings, nozzle condition, material consistency and extrusion rate. No confirmed solution is recorded; it is to be verified with the operator or technician. Safety / Important Notes: Do not present any of the above as a confirmed diagnosis. Related but distinct issues, which must be kept separate: Delamination, and Poor Layer Bonding. Important Parameters: None specified in the source. Verification Status: UNVERIFIED - visual observation only. Confirmed root cause: NOT CONFIRMED. Confirmed solution: NOT CONFIRMED. Source: PDF 2, Section 3. English Question Prompts: [Q346] Why is there a gap in my printed layer? [Q347] What causes voids in the wall? [Q348] There is a hole in the printed layer. Layer gap or void.? Hinglish Question Prompts: [Q349] Layer mein gap aa raha hai. Wall mein void dikh raha hai. Material properly fill nahi ho raha.?

Record #89: Issue: Layer Deformation or Irregular Deposition Problem: An irregular or deformed printed bead or layer is visible, especially around a corner. Symptoms / Observations: Some layers do not maintain the same shape or profile as the surrounding layers. Cause / Confidence: Possible causes include excessive extrusion, incorrect nozzle height, incorrect print speed, material that is too wet or soft, or motion changes at corners. The root cause is NOT confirmed from the images alone. Solution / Corrective Action: Troubleshooting to verify - check extrusion rate, pump pressure, nozzle height, print speed, material consistency and corner movement. No confirmed solution is recorded; it is to be verified with the operator or technician. Safety / Important Notes: Do not present any of the above as a confirmed diagnosis. Important Parameters: None specified in the source. Verification Status: UNVERIFIED - visual observation only. Confirmed root cause: NOT CONFIRMED. Confirmed solution: NOT CONFIRMED. Source: PDF 2, Section 4. English Question Prompts: [Q350] The layer is deformed. Why is the bead irregular at corners? [Q351] The layer shape is wrong.? Hinglish Question Prompts: [Q352] Layer ka shape kharab hai. Corner par bead irregular aa raha hai. Layer deform ho gayi.?

Record #90: Issue: Vertical or Through-Layer Crack Problem: A prominent vertical crack is visible across multiple printed layers. Symptoms / Observations: The crack crosses several horizontal printing layers. Cause / Confidence: Possible causes include poor inter-layer bonding, material drying or shrinkage, excessive time gap between layers, material moisture and workability variation, or insufficient adhesion. The root cause is NOT confirmed from the images alone. Solution / Corrective Action: Troubleshooting to verify - check layer-to-layer bonding, material condition, printing interval, environmental conditions, and mix workability. No confirmed solution is recorded; it is to be verified with the operator or technician. Safety / Important Notes: Do not present any of the above as a confirmed diagnosis. For the site-verified action on cracks, see the site-incidents record "Cracks in Printed Wall", which specifies filling the crack before shifting and inspecting print and material conditions. Important Parameters: None specified in the source. Verification Status: UNVERIFIED - visual observation only. Confirmed root cause: NOT CONFIRMED. Confirmed solution: NOT CONFIRMED. Source: PDF 2, Section 5. English Question Prompts: [Q353] There is a vertical crack through the layers. Why does a crack cross several layers? [Q354] Through-layer crack.? Hinglish Question Prompts: [Q355] Wall mein vertical crack aa gaya hai. Crack kayi layers ke aar paar hai.?

Record #91: Issue: Fine Surface Cracking Problem: Fine cracks are visible on the upper printed surface. Symptoms / Observations: Small surface cracks are visible along the highlighted section. Cause / Confidence: Possible causes include rapid drying, shrinkage during setting, material mix and workability variation, heat, wind, or other environmental conditions. The root cause is NOT confirmed from the images alone. Solution / Corrective Action: Troubleshooting to verify - check material consistency, moisture and workability, layer interval, and curing and environmental conditions. No confirmed solution is recorded; it is to be verified with the operator or technician. Safety / Important Notes: Do not present any of the above as a confirmed diagnosis. Important Parameters: None specified in the source. Related site instruction: printed walls were to be cured three times per day. Verification Status: UNVERIFIED - visual observation only. Confirmed root cause: NOT CONFIRMED. Confirmed solution: NOT CONFIRMED. Source: PDF 2, Section 6. English Question Prompts: [Q356] There are fine cracks on the surface. Why is the top surface cracking? [Q357] Small surface cracks.? Hinglish Question Prompts: [Q358] Surface par fine cracks aa rahe hain. Upar ki surface phat rahi hai.?

Section 7.8: Kelvin6K Core Operating Manual (SOPs) (kelvin6k_manual.md)

Record #92: Nozzle Clog Procedure Procedural Content: If material flow drops or the layer becomes too narrow, the nozzle may be clogged. Stop the pump, release system pressure, then soak the nozzle in solvent for ten minutes and clear it with the cleaning rod. Never clear a clog while the pump is pressurised.

Record #93: Layer Width Calibration Procedural Content: Layer width is measured in pixels by the vision system and converted using PIXELS_PER_MM. Place a reference block of known width at the print distance, count the pixels it spans, and divide by its width in millimetres.

Record #94: Daily Maintenance Procedural Content: Clean the printing pipe every four hours of operation. Inspect the accelerator coupling daily and check pump seals weekly.

Section 7.9: Kelvin6K Factory Troubleshooting Guide (kelvin6k_troubleshooting.md)

Record #95: Delamination Procedural Content: Delamination is the separation of adjacent printed layers, where a layer fails to bond to the one beneath it and a visible split or gap forms along the layer line. In large-format concrete printing it is usually caused by too long an interval between layers, so the lower layer has cured too far before the next is placed; by a dry or poorly mixed material batch; or by low ambient humidity and high temperature accelerating surface drying. Delaminated walls lose structural strength and must be inspected before further layers are added.

Record #96: Layer Shifting Procedural Content: Layer shifting is a horizontal offset between one layer and the next, so the wall leans or steps rather than rising vertically. Common causes are mechanical: a loose drive belt or coupling, a skipped step on a drive motor, an obstruction in the gantry path, or the printed part being knocked while still green. It can also be caused by an unstable base layer that slumps under the weight of layers above. Uneven layer pitch measured by the vision system is an early indicator.

Record #97: Poor Layer Bonding Procedural Content: Poor layer bonding happens when adjacent layers touch but do not fuse into a continuous section. Typical causes are insufficient material flow at the nozzle, too fast a print speed for the extrusion rate, a layer interval that is too long, contamination or dust on the previous layer surface, and incorrect accelerator dosing. Bonding problems often appear first as reduced bead width, so a narrowing layer width trend should be investigated promptly.

Record #98: Over-extrusion and Wide Layers Procedural Content: A layer that is wider than the configured maximum indicates over-extrusion: too much material for the travel speed. Causes include pump speed set too high, accelerator dosing too low so the material stays fluid, or the nozzle standing off too close to the previous layer. Excessively wide layers slump under load, distort wall geometry, and waste material.

Record #99: Under-extrusion and Narrow Layers Procedural Content: A layer narrower than the configured minimum indicates under-extrusion. Causes include a partially clogged nozzle, low material in the hopper, a failing pump seal, or air entrained in the delivery line. Narrow layers reduce wall thickness and weaken the structure.

8. Identification of Metadata & Source-Only Chunks

In heading-aware RAG chunking, file-level preambles (headers, document titles, and citation lines) that appear before the first substantive section header are placed into initial chunks (Chunk 0). The following two chunks in the index contain only document titles and zero operational procedures:

• kelvin6k_manual.md (Chunk 0): # Kelvin6K Operating Manual - Contains only the top-level document title without operational text. If retrieved, speak_knowledge() loops to Chunk 1 (Nozzle Clog Procedure) to deliver procedural content.

• kelvin6k_troubleshooting.md (Chunk 0): # Kelvin6K Troubleshooting Guide - Contains only the guide title. speak_knowledge() loops to Chunk 1 (Delamination) or subsequent diagnostic sections.

• operational_rules.md (Chunk 0 Header): Sources: 3D_Printing_Issues_Solutions_AI_Agent_Dataset_No_Atom_V2(3).pdf... - Contains source credits. Filtered by speak_record() via case-insensitive prefix matching on 'Sources:' to prevent file citations in spoken voice responses.

9. Knowledge Gaps & Out-of-Scope Topics Analysis

To prevent operator confusion and ensure safety on the construction site, the following domains are identified as completely absent from the Kelvin6K RAG knowledge base. When asked about these topics, the assistant must return 'That information is not in the available documentation' rather than attempting to interpolate answers.

• 1. Polymer / FDM 3D Printing Concepts: There is ZERO knowledge regarding plastic filaments, heated print beds, bed leveling screws, thermal warping, infill geometric patterns, or retraction distance/speed. Kelvin6K is an industrial mortar/concrete deposition system.

• 2. Real-Time Telemetry & Sensor Values: The RAG contains documented procedures and historical site records, but NOT live telemetry. Instantaneous pump line pressure, live motor thermals, live robot XYZ coordinates, and live visual feeds are accessed via FastAPI /api/ status endpoints, not RAG.

• 3. Concrete Mix Chemistry & Admixture Formulations: The knowledge mentions raw materials (sand, fly ash, accelerator) and physical handling, but contains NO exact chemical formulations, water-to-cement ratios, accelerator chemical brand dosages, or cementitious slump design charts.

• 4. Slicer Software Menus & Toolpath Generation: There is no documentation on slicing software GUI menus, CAD file preparation, or custom G-code generation algorithms beyond high-level notes regarding file naming conventions and feedrate overrides.

• 5. Civil Engineering Building Codes & Structural Certifications: The RAG contains no structural load-bearing building code certifications, seismic calculations, or civil structural engineering approval standards.

• 6. Confirmed Root Causes for Visual Camera Defects: In visual_defects.md, root causes are explicitly documented as 'Not confirmed from the images alone' and solutions as 'To be verified with operator/technician'. The assistant must not present them as definitive.

10. What Kelvin6K Voice Assistant Can Answer

Kelvin6K is fully equipped to act as an on-site, hands-free operational co-pilot for 3D concrete printing operators. Operators can speak naturally in English or Hinglish using the wake-word 'Okay Kelvin' followed by their operational query. The table below summarizes the operational domains Kelvin6K can immediately answer with verified procedural guidance.

| Operational Domain | Example Voice Questions (English / Hinglish) | Delivered Procedural Response |
| --- | --- | --- |
| Cleaning Schedules & Tools | 'How often do I clean the accelerator hole?' 'Pipe kitne ghante mein clean karna hai?' | Clean hole every 5 layers with big Allen key. Clean printing pipe every 4 hours. Clean hopper after each wall. Clean pump front after 2 walls. |
| Pumping & Hose Failures | 'Material is not coming out of hose.' 'Hose repeatedly choke ho raha hai.' | Stop pump, depressurize line, check for hardened batch, tap hose to locate blockage, remove stones from pump stator, check sand screening. |
| Nozzle Blockages & Clogs | 'Nozzle clog procedure kya hai?' 'Why is material delivery inconsistent?' | Stop pump, release system pressure, soak nozzle in solvent for 10 minutes, clear with cleaning rod. Never clear while pressurized. |
| Hardware Alarms & Faults | 'How do I fix Z-axis servo fault E08.5?' 'DC startup issue aa raha hai.' | Inspect J1 ethernet cable, check 24V power supply, cycle distributed clock power, check VFD wiring and loose terminal connections. |
| Print Quality & Defects | 'How do I prevent delamination?' 'Layer thickness uneven aa rahi hai.' | Shorten inter-layer cycle time, verify accelerator dosing, ensure prior layer surface is moist and free of dust, adjust travel speed. |
| Environmental & Safety | 'Can we continue printing in the rain?' 'Pump is in direct sun.' | Stop printing immediately upon rain, cover electricals and hopper with tarpaulin. Shade pump to prevent premature concrete setting. |
| Ambiguities & Standards | 'What is the correct layer thickness?' 'Feedrate 150 percent ka kya matlab hai?' | States both values clearly: 40 mm per operating manual vs 50 mm per site observation. Directs operator to confirm with shift engineer. |